# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2010

# Subject Title:	FLUID MECHANICS AND MACHINERY     			      Time: 3 hours

# Subject Code:	09CE242				       					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Capillary rise is caused by _________ forces.

2.	On a free surface of a liquid the pressure is _________.

3.	Free vertex flow is irrotational. (True/False)

4.	Bernoulli equation is applicable for flows which are _________.

5.	A V notch is used to measure _________.

6.	Minor losses are losses due to _________.

7.	The shut off head of a centrifugal pump is _________.

8.	The angle of deflection of the jet in the case of pelton turbine is about _________.

9.	What is Flow ratio?

10.	Define specific speed.

PART – B (5 x 3 = 15 MARKS)

11.	A circular disc of 0.3 m diameter rotates over a large stationary plate with 1 mm thick fluid film between them. Determine the viscosity of the fluid if the torque required to rotate the disc at 300 rpm was 0.1 Nm.

12.	In a two dimensional flow the x and y directional velocities u and v are given by

- Show that the flow is steady 	b.	Check whether the flow is irrotational

13.	A pitot static tube is mounted on an aircraft travelling at a speed 300 kmph against a wind velocity of 20 kmph. If the specific weight of air is 12 N/m3 , determine the pressure difference the instrument will register.

14.	A jet 30 mm diameter with velocity of 10 m/s strikes a vertical plate in the normal direction. Determine the force on the plate if (i) The plate is stationary (ii) If it moves with a velocity of 4 m/s towards the jet.

15.	What is cavitation? How does it affect the performance of hydraulic machines?

PART – C (5 x 15 = 75 MARKS)

16.	a.	An inverted U-tube manometer is connected to two horizontal pipes A and B through 	which water is flowing. The vertical distance between the axes of this pipe is 30 cm. 	When 	an oil of specific gravity 0.8 is used as a gauge fluid the vertical heights of water 	columns in the two limbs of the inverted manometer are found to be same and equal to 35	cm. Determine the difference of pressure between the pipes.						(8)

b.	An oil of viscosity 5 poise is used for lubrication between a shaft and sleeve. The diameter of the shaft is 0.5 m and it rotates at 200 r.p.m. Calculate the power lost in oil for a sleeve length of 100 mm. The thickness of oil film is 1.0 mm.							(7)

(OR)

[P.T.O]

17.	a.	Derive an expression for the force exerted on a sub-merged vertical plane surface by the      	static liquid and locate the position of center of pressure.							(7)

b.	Determine the total pressure and center of pressure on an isosceles triangular plate of base 4 m and altitude 4 m when it is immersed vertically in an oil of specific gravity 0.9. The base of the plate coincides with the free surface of oil.							(8)

18.	Derive Bernoulli’s equation from Euler’s equation of motion and state the assumptions made for such a derivation.

(OR)

19.	a.	Derive an expression for continuity equation for three-dimensional flow.			(7)

b.	Define the terms

i.	Total acceleration 	ii.	Convective acceleration 	  	iii.	Local acceleration

iv.	Stream function.															(8)

20.	Derive Darcy-Weisbach Equation and chezy’s formula to find the loss of head due to friction in pipes.

(OR)

21.	a.	An orifice meter with orifice diameter 10 cm is inserted in a pipe of 20 cm diameter .The 	pressure gauges fitted upstream and downstream of the orifice meter gives readings of 	19.62N/cm2 and 9.81N/cm2 respectively. Co-efficient of discharge for the meter is given 	as 0.6. Find the discharge of water through pipe.									(8)

b.	Explain the principle of venturimeter with neat sketch. Derive the expression for the rate of flow of fluid through it.														(7)

22.	A centrifugal pump having outer diameter equal to two times the inner diameter and running at 1000 rpm works against a total head of 40 m. The velocity of flow through the impeller is constant and equal to 2.5 m/s. The vanes are set back at an angle of 40o at outlet .If the outer diameter of the impeller is 500 mm and width at outlet is 50 mm determine (i) Vane angle at inlet,  (ii) work done by impeller on water per second and   (iii) manometric efficiency.

(OR)

23.	Derive an expression for work done per second per unit weight of fluid striking per second on a series of moving curved vanes at one of the tips of the vane with neat sketch of velocity triangle diagram.

24.	The jet velocity in a pelton turbine is 65 m/s. The peripheral velocity of the runner is 25 m/s. The jet is deflected by 160° by the bucket. Determine the power developed and hydraulic efficiency of the turbine for a flow rate of 0.9 m3/s.  Also determine the number of jets required. If the turbine produce 15 MW power under a head of 480 m when running at 500 rpm, Take wheel diameter to jet diameter ratio is 10. Assume ηo = 85%, Cv = 0.97, φ = 0.46

(OR)

25.	a.	Describe working of vane pump and gear pump with neat sketches.

b.	A turbine is to operate under a head of 25m at 200rpm. The discharge is 9m3/s .If the   efficiency is 90% , determine specific speed of the machine and power generated.