# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2010

# Subject Title: 	COMPUTATIONAL FLUID DYNAMICS				Time : 3 hours

# Subject Code: 	10ME402		         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Derive continuity and momentum equations for reactive flow.

2.	Solve the Laplace equation,

2u = 0;  0  x  , 0  y

subjected to boundary conditions

u(x, 0) = sinx + 2 sin 2x, u(,y) = 0, u(x, ) = 0, u(0, y)  = 0

(OR)

3.	Classify the behavior of the following system of PDEs in (t, x) and (t, y) space and transform the PDEs to canonical form

;

4.	A plane wall of thickness 6 cm and k = 5 W/mK generated heat at a uniform rate                          S = 100 kW /m3.   Both sides of the wall are experiencing convection with air at 10oC,               h = 100 W/m2K.  Find the steady state temperature distribution in the wall using 7 equally spaced nodes.

(OR)

5.	A square plate of size 1 x 1 is initially at a uniform temperature T = 0. For times t > 0, one of its edges is brought to T = 1, whereas the remaining edges are kept T = 0.  Using implicit formulation with x = y = 1/3 and t= 0.1.  Find T(x, y, t) for an elapsed time of t =0.3.

6. 	Use 7 cells with L= 0.7, F = u = 1 and  = 1 to solve one dimensional convection diffusion equation

Using, (a) Central difference    (b)  Upwind scheme

(OR)

7.	Derive the exact solution for convection diffusion equation with no heat generation.  What happens when Pe  0 and  ?

8.	Irrotational flow in a duct of variable shape is shown in the figure.  AF = 0.5, BC = 1.0,       EF = 1.0, CD = 1.0.  Inlet and outlet boundaries are AF and BC respectively.

Q=4

A

F

E

D

C

B

Q=4

Use a uniform square grid of size 0.25 x 0.25 to find the distribution of

(OR)

9.	Write the steps involved in SIMPLE algorithm.