# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2010

# Subject Title:	NETWORK THEORY 									      Time: 3 hours

# Subject Code:	EE246		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Express the neutral current in terms of line currents in an unbalanced 3Φ, 4-wire star connected system.

2.	Three equal resistances of 5 Ω are connected in delta circuit. Obtain the equivalent star circuit.

3.	What is the significance of dot convention in coupled circuits?

4.	Define coupled circuits.

5.	Define time constant.

6.	Draw the equivalent circuit for capacitor in Laplace domain.

7.	Define poles and zeros of network function.

8.	Which parameters are useful in characterizing cascaded two-port networks?

9.	Define: Foster reactance theorem.

10.	What are the two basic forms used for network realization?

PART – B (5 x 3 = 15 MARKS)

11.	A 400V, 3Φ, 50Hz balanced supply is applied to a three star connected load of 4+j3Ω  per phase .The phase current is 12A . Find the total apparent power.

12.	Two 100µH coils have a mutual inductance of 50µH.  Determine the total inductance when they are connected in series aiding connection.

13.	Find ABCD parameters of the given ideal transformer circuit shown in Fig.

14.	What are the chief properties used to characterize a passive filter?

15.	Check whether the polynomial Ps=s3+6s2+11s+6  is Hurwitz or not.

PART – C (5 x 15 = 75 MARKS)

16.	a.	A 400V, 3Φ, 50Hz balanced supply is applied to a three phase load and two wattmeters are 	connected to measure the power. The two wattmeter readings are +15kW and-4kW. 	Determine the true power drawn by the load, power factor and line current. 			(8)

b.	Three identical resistances are connected in a star fashion against a balanced three phase voltage supply. If one of the resistances be removed, by how much is the power reduced?

(7)

(OR)

17.	A 400V, 3Φ, 50Hz balanced supply is connected to an unbalanced four -wire star connected load. The branch impedances of the load are ZR =4+j8Ω  ;ZY =3+j4Ω  and

ZB =15+j20Ω  . Find the (a) line currents, (b) current in the neutral wire and (c) the total power.

[P.T.O]

18.	a.	Find an expression for the input impedance of the given coupled circuit.			(10)

b.	Find the self and mutual inductances for two coupled coils N1 = 500 turns, N2 = 750 turns, I1 = 2A, 1 = 10mWb and 	12 = 6m	Wb.											(5)

(OR)

19.	Obtain the expression of maximum amplification/ maximum output voltage in double-tuned coupled circuits. Also explain the variation of output voltage with frequency for different coupling co-efficient values.

20.	Determine the transient current when the switch is moved from the position 1 to position 2 at
t = 0 in the given figure. The circuit is in steady state with the switch in position 1. The voltage applied to the circuit is V(t) = 150 cos (200t + 30o) volt.

(OR)

21.	A step voltage V(t) = 100u(t) is applied in a series RLC circuit shown in figure. When the switch is closed at t = 0, obtain the expression for the transient current.

22.	a.	Obtain transmission parameters in terms of short circuit admittance parameters. 		(7)

b.	A symmetrical T-section has the following open circuit and short circuit impedances:

ZOC = 800, ZSC = 600.	Determine T-section parameters to represent the two-port network.                            														(8)

(OR)

23.	Obtain the open circuit parameters of the bridged T-network shown in Figure.

24.	Find the positive realness of the given function	.

(OR)

25.	The driving point impedance function of an LC network is given by 	Obtain the first and second Cauer forms.