# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2010

# Subject Title:	DIGITAL CONTROL SYSTEMS 						      Time: 3 hours

# Subject Code:	EI259		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is the disadvantage of weighted resistor DAC?

2.	What is the function of holding device?

3.	State final value theorem in Z transforms.

4.	Obtain the Z-transform of   6 δ(k-2) + δ(k)

5.	Represent the unit delayer in Z-domain.

6.	Give any one advantage of parallel realization.

7.	How to map the points on imaginary axis of s-plane into z-plane?

8.	What do you mean by truncation?

9.	What is ringing in digital controller?

10.	State the condition when a controller is said to be physically unrealizable.

PART – B (5 x 3 = 15 MARKS)

11.	Define acquisition time, aperture time and droop rate with respect to sample and hold operation.

12.	Obtain the inverse Z-transformation of the function   G(z) = .

13.	Z 3( Z – 0.333) ( Z-1 )Define controllability and observability.

14.	What is meant by digital quantization?

15.	Briefly discuss the design specifications of dead beat algorithm.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive the transfer function of Zero Order Hold.									(8)

b.	Briefly explain the operation of R-2R ladder 3-bit D/A converter with neat circuit.	(7)

(OR)

17.	a.	Explain the principle and operation of Dual slope type A/D Converter with neat diagram.

(8)

b.	Describe in detail about the issues in selecting the sampling frequency.				(7)

18.	a.	Obtain the modified z-transform of unit ramp function.							(5)

ZOH

C(s)

R(s)

T=1 Sec

T

+

-

b.	For the sampled data system given below, find the response c(k) for unit step change in input r(k)																	(10)

(OR)

[P.T.O]

19.	a.	 Find the pulse transfer function Y(z)/ X(z) for the system given below.				(10)

e-3s

Y(S)

X(S)

118s+1

T=1.2 Sec

b.	If X(z) = 2+3z-1+ 4z-2 find initial and final values of x(k).							(5)

20.	Obtain the direct, cascade and parallel realization for the given pulse transfer function.

C(z)R(z)= z+2z2+5z+3

(OR)

21.	Obtain the phase variable form of state model of the following system and find the controllability of the same.

y(k+3) + 6y(k+2) +  11y(k+1) + 6y(k) =U(k)

22.	Find state transition matrix Ф(k) if the system state matrix is given as  A= 01-127.

(OR)

23.	Consider the sampled data system shown in following figure. For a sampling period 0.25 second, determine the value of ‘K’ such that the system is stable.

T=1 Sec

Ks(s+4)

C(s)

R(s)

24.	a.	Derive the digital equivalent of position form of PID algorithm.					(8)

b.	Explain any digital control system using microcontroller with block diagram.		(7)

(OR)

25.	Design Deadbeat controller algorithm for a system having a model Gp(s)=5e-2s0.3s+1 with      sampling period equal to 2 sec. Also check whether the controller is physically realizable or not.