# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	AIRCRAFT STRUCTURES 								       Time: 3 hours

# Subject Code:	09AE202	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	________ is the line of cross section about which product moment of inertia zero.

2.	In a thin walled structure of length ‘L’, the relation between the shear flow and angle of twist subjected to torque is ________.

3.	To carry a load moncoque structure is________ than semi-monocoque structure.

4.	The part of the wing that reacts to the combined (bending and torsional) loading is _________.

5.	A structure has number of joints as 6 and numbers of members as 10, then the structures is said to be ________.

6.	The stiffness factor equation for the fixed hinged case of beam is ________.

7.	The strain energy stored by the body within the elastic limit is called ________ .

8.	Strain energy equation for a cantilever beam of length ‘L’ subjected to tip load ‘W’ is________

9.	According to Euler’s theory, the material of the column is ________

10.	Stress difference theory is applicable for ________ materials.

PART – B (5 x 3 = 15 MARKS)

11.	Define Shear center and locate the same for ‘L’ and ‘T’ sections.

12.	Write down the three purpose of frames (rings) structure of the fuselage.

13.	Write down the Claypeyron’s three moment equations

a.	When all the supports remain at the same level.		b.	For sinking of supports.

14.	Explain the Castigliano’s theorem with its formula.

15.	Write down the Euler’s formula for different end conditions.

PART – C (5 x 15 = 75 MARKS)

16.	A beam having the cross section shown in figure is subjected to a bending moment of 1500 Nm in a vertical plane. Calculate the maximum direct stress due to bending and also state the point at which it acts.

(OR)

[P.T.O]

17.	a.	Explain the Bredt-Batho theory.													(5)

b.	A two cell tube shown in figure subjected to a torque 100kN.  Calculate the shear flow values in each cell and draw the shear flow diagram.										(10)

18.	Determine the shear flow in the web of the tapered beam shown in figure at a section midway along its length. The web of the beam has a thickness of 2mm and is fully effective in resisting direct stress. The beam tapers symmetrically about its horizontal axis and the cross sectional area of each flange is 400mm2.

(OR)

19.	A cantilever beam carries a concentrated load as shown in figure. Calculate the distribution of stiffner load and shear flow distribution in the web panels. Assume that the given webs are effective in shear.

[P.T.O]

20.	An inclined truss loaded as shown in figure. Determine the forces of the member by nay one method.

(OR)

21.	A continuous beam ABCD consists of three span and is loaded as shown in figure. Find the supported end moments and bending moment diagram using Moment distribution method.

22.	a.	A cantilever beam of 8m subjected to loading as shown in figure. Determine the reaction at 	the prop end using castigliano’s theorem.											(7)

b.	State and prove the Maxwell’s Reciprocal theorem.									(8)

(OR)

23.	The cross sectional area of the each member of the truss shown in figure. A=400mm2 and E=200GPa. Based on Unit load method

a.	Determine the vertical displacement of joint C if a 4kN force is applied to the truss at C.	(10)

b.	If no load acts on the truss, what would be the vertical displacement of joint C if a member AB 	were 5mm too short?					 											(5)

[P.T.O]

24.	a.	Derive the Euler’s formula for one end of the column is fixed and other end pinned condition.						             		         												(10)

b.	Calculate and find the ratio between the Rankine’s and Euler’s crippling loads formula for tubular strut 2.25m long having outer and inner diameter of 37.5mm and 32.5mm loaded through pin jointed at both ends. Given: yield compressive stress as 315 MN/m2; a= 1/7500; E=200GN/m2. If the Euler’s stress is 200MN/m2 find the equivalent length of the column also.

(5)

(OR)

25.	A circular shaft of tensile yield strength 260MPa is subjected to a combined state of loading defined by a bending moment M=15kN-m and Torque T= 20kN-m. Calculate the diameter of the shaft by applying the following theories.

a.	Maximum shear stress theory														(5)

b.	Strain energy theory																(5)

c.	Shear strain energy theory															(5)