# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title : 	AERODYNAMICS  										      Time : 3 hours

# Subject Code:	09AE205	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The direction of Drag is

a.	Along fuselage axis					b.	Normal to fuselage axis

c.	In the direction of Velocity vector		d.	Normal to the direction of Velocity vector

2.	The direction of Lift is

a.	Along fuselage axis					b.	Normal to fuselage axis

c.	In the direction of Velocity vector		d.	Normal to the direction of Velocity vector

3.	Incompressible flow is indicated by constant

a.	Pressure								b.	Temperature

c.	Density

4.	Incompressible flow in a pipe is an example of  ____________ flow

5.	The two major components of the drag are ________ drag and ________ drag.

6.	The efficiency of the propeller changes with free stream velocity as follows:

a.	Continuously increases as free stream velocity increases.

b.	Continuously decreases as free stream velocity increases.

c.	Initially increases and then decreases as free stream velocity increases.

d.	Initially decreases and then increases as free stream velocity increases.

7.	The Service Ceiling of the aircraft is the altitude at which the maximum rate of climb is

a.	100m/s						b.	30m/s

c.	100km/hr					d.	30km/hr

8.	Cruise flight consist of

a.	Un-accelerated, straight and horizontal flight, with no bank or yaw

b.	Straight, horizontal and accelerated flight

c.	Accelerated and Horizontal fight with no bank or yaw

d.	Accelerated and Horizontal fight

9.	Instruments used for measurements of flow velocity are ________ and ________ Doppler anemometer.

10.	For measurement of force on a model in a wind tunnel, one normally uses _________balance.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Pitching Moment. With the help of proper figure, explain the significance of its sign and its significance.

12.	Define Doublet and its strength. Draw streamlines for flow due to a doublet.

13.	Explain the method for estimating the drag of a complex body having many components like aircraft.

14.	Define range and endurance.

[P.T.O]

15.	In Supersonic Wind Tunnel, Stagnation pressure and Stagnation Temperature are 1000kPa and 300K respectively. When the flow expands to Mach 3 in the test section, what is the pressure and temperature in the test section. (Assume  γ = 1.4)

PART – C  (5 x 15 = 75 MARKS)

16.	Define Drag. How can the drag of a vehicle be estimated from the known pressure and Shear stress distribution. Derive the expression for drag and drag coefficient of a 2D body at angle of attack 0o as function of pressure and shear stress distribution.

(OR)

17.	Using Pi-Buckingham theorem, derive the non-dimensional parameters which determine the force on a body in viscous flow.

18.	Estimate the lift of rotating cylinder in uniform cross flow stream. The density of flow is ρ, free stream velocity U∞, Radius of cylinder R and angular speed Ω.

(OR)

19.	From the fundamental principles, derive continuity equation in cylindrical coordinates system.

20.	Explain the mechanism of thrust generation for a propeller engine.

(OR)

21.	For a rocket motor, derive the following expression for thrust for flow through Convergent-Divergent nozzle in terms Total pressure Po, Exit pressure Pe, ratio of specific heat γ, Throat area A*, Exit area Ae. Assume that nozzle is choked that is at throat Mach number is unity(1.0).

T= A*po2γ+1γ+1γ-12γ2γ-11- pepoγ-1γ+ Aepe- p∞

22.	Consider an aircraft with the following specification:

Total weight			:	12500 N

Wing Area			: 	6.12 m2

Maximum Thrust		: 	2600 N

CDo					:	0.032

β					:	0.1

CLmax				:	1.22

For sea level flight ( sea level air density 1.225 kg/m3), find the climb rate when aircraft is flying at an speed of 100m/s.

(OR)

23.	Consider a turbojet aircraft whose wing loading is 4000N/m2, TSFC is 0.8/hr, and whose cruise-fuel weight fraction is 0.3 during its cruising flight (Weight of fuel is 30% of the starting weight of the aircraft). The aspect ratio of its wing is 7.2 and the Oswalds span efficiency factor e is 0.85. The zero-lift drag coefficient of the aircraft is 0016. The aircraft started cruising flight at an altitude of 9 km with an airspeed of 800 krn/h. Calculate the range, and endurance of the aircraft flight in standard atmosphere for the cases of constant h-CL

At the altitude of 9km the value of σ = (ρ/ρSL) = 0.3813 as obtained from the standard atmospheric table. Sea level density of air is 1.225 kg/m3.

Note:  CD= CDo+ β CL2

where β= 1e π AR

24.	Explain in detail the principle of hot-wire anemometry.

(OR)

25.	What are the different steps involved in wind tunnel testing? Explain each step briefly.