# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	AIRCRAFT PERFORMANCE 							       Time: 3 hours

# Subject Code:	09AE216	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	In four digit series aerofoil, the last two digits describes the ________ of airfoil in percentage of chord.

2.	Wave Drag is produced at subsonic speeds. (True/False)

3.	At high Reynolds number ________ drag is more.

4.	Winglets are used to reduce ________ Drag.

5.	Absolute ceiling is the altitude at which Rate of Climb is ________.

6.	TSFC is defined as ________.

7.	The commonly used thrust augmentation in fighter aircraft is ________.

8.	An altitude at which the rate of climb of an aircraft is 100 ft/min is called as ________.

9.	The measure of power absorbing capacity of the propeller is called as ________.

10.	Fixed pitch propeller produces maximum performance in all altitudes. (True/False).

PART – B (5 x 3 = 15 MARKS)

11.	Explain the 7 Digit NACA Numbering systems.

12.	What is called induced drag?  How is it reduced?

13.	What is the effect of altitude on the power required for a steady level flight?

14.	Define Range and Endurance.

15.	What are the basic assumptions in Froude momentum theory?

PART – C (5 x 15 = 75 MARKS)

16.	Obtain expression for lift when flow passes a rotating cylinder.

(OR)

17.	Explain the NACA aerofoil numbering system.

18.	a.	Explain the principle of boundary layer control in aircraft drag reduction.   			(5)

b.	Write notes on various types of drag acting on an airplane.		     				(10)

(OR)

19.	An aircraft weighs 250 kN has a wing area of 80 m2 and its drag equation is.  Calculate the (a) Minimum thrust required and (b)  Minimum power required for straight and level flight and the corresponding true air speed at sea level at an altitude where  Assume . Also derive the equation used.

20.	Derive the conditions for minimum drag and minimum powered required for a jet powered aircraft.

(OR)

21.	a.	Derive the Brequette Range formula for Jet-Propelled airplanes.

b.	Discuss some methods of thrust augmentation.

[P.T.O]

22.	Show that for a propeller driven airplane the maximum rate of climb is given by

Where =propeller efficiency

P=Brake Horse power

Also discuss its implications.

(OR)

23.	Based on Froude’s momentum theory, derive an expression for the ideal propulsive efficiency of an airscrew.

24.	Obtain the expression for turn radius and turn rate for pull up and pushover manoeuvre.

(OR)

25.	Write notes on the following:

a.	High lift devices.																(8)

b.	Types of propellers 															(7)