# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	PRINCIPLES OF CHEMICAL ENGINEERING 			       Time: 3 hours

# Subject Code:	09BT203	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Ratio of gram moles of solute to mass of solvent in kg is defined as __________.

2.	For NTP conditions at 273.15K and 101.325kPa volume occupied by 1kmol of gas is _______.

a.	 22.4 m3/sec		b.	20.4  m3/sec		       c.	2.24 m3/sec.

3.	A stream bled off to remove an accumulation of certain inerts or unwanted materials that might otherwise build up in recycle stream is called __________.

4.	Operation used in industry for concentration of weak liquor to produce thick liquor is _______.

a.	Evaporation		b.	Drying		c.	Crystallisation

5.	Stoke (St) is the representation of __________in cgs system.

6.	Friction generated in unseparated boundary layer is __________.

7.	When fluid flows through a pipe line maximum velocity occurs at the walls of the pipe line (True /False).

8.	Area flow meter is __________.

a.	Rotameter 	b.	Orifice meter 	c.	Venturi meter

9.	To avoid cavitation the pressure at the pump inlet must exceed the vapour pressure by a certain value __________.

10.	Velocity head of the liquid from the impeller is converted into pressure head in __________ pump.

a.	Centrifugal  		b.	Rotary		c.	Plunger  	d.	Diaphragm.

PART – B (5 x 3 = 15 MARKS)

11.	State Simpson’s rule and elucidate its application.

12.	Write the general and overall material balance with block diagram for a

a.	Distillation unit			b.	Extraction unit

13.	Discuss in detail about the rhelogical behavior of non-newtonian fluids.

14.	Differentiate venturimeter and orifice meter.

15.	Explain the performance characteristics of a pump working with centrifugal force.

PART – C (5 x 15 = 75 MARKS)

16.	A gas mixture has the following composition by volume:

SO2= 8.5%, O2=10% and N2=81.5%

Find:	a.	Density of the gas mixture at a temperature at 473 K and 202.65kPa

b.	Composition by weight

Data: Molecular weight on SO2=64, O2=32, N2=28.

(OR)

[P.T.O]

17.	a.	An aqueous solution of sodium chloride is prepared by dissolving 25 kg of sodium	chloride in 100 kg of water. Determine (a) Weight % and (b) mole % composition of 	solution.		Molecular weight of NaCl = 58.5									(10)

b.	Assuming air to contain 79% N2 and 21% O2 by volume, calculate the density of air at 	NTP.                                                                                      						(5)

18.	Wet solid containing 60% water and 40% solids are to be dried in a dryer to get solids containing 8% water by weight. The fresh air containing 0.015 kg water vapour per kg dry air and air leaving the dryer contains 0.06kg water vapour per kg dry air. If the quantity of air entering the dryer per kg of solids (dry) is 100kg.  Calculate.

a.	The quantity of fresh air		b.	The fraction of air recirculated and

c.	The recycle ratio

(OR)

19.	In the Deacon process for the manufacture of chlorine a dry mixture of hydrochloric acid 	gas and air is passed over a heated catalyst which promotes oxidation of acid.   Air is used 30% in excess of that theoretically required. kg of the acid. (Atomic weight of chlorine = 35.5, air contains 23.2% oxygen by weight)

Reaction : 4 HCL + O2  → 2Cl2 + 2H2O.

Calculate the following:

a.	The weight of air supplied per kg of acid.

b.	The composition (weight %) of gases entering.

c.	The composition (weight %) gases leaving, assuming that 60% of acid is oxidized in 	the 	process.

20.	Derive and discuss in detail the mechanical energy equation with its corrected form for fluid friction.

(OR)

21.	a.	Discuss in detail about head losses for various stations when fluid flows through a pipe     		line.                                                                                       						(10)

b.	Derive barometric equation for estimating the pressure distribution.            			(5)

22.	Derive Ergun equation for fluid flows through packed bed.

(OR)

23.	a.	With a neat sketch differentiate gate valve and globe valve.  						(9)

b.	Discuss the common device used for preventing the leakage around moving parts.	(6).

24.	Write short notes on:

a.	NPSH and Cavitation		b.	Positive displacement compressor		c.	Priming.

(OR)

25.	With a neat explain the construction, performance and working principle of centrifugal pump.