# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April /May 2011

# Subject Title:	PROCESS ENGINEERING CALCULATIONS 			       Time: 3 hours

# Subject Code:	CL207		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The reactants present in excess is termed as _________.

2.	The ratio of the volume of each component and the total volume of the system, for each 100 part of the total volume is called as _________.

3.	Dalton`s law of partial pressures P = _________.

4.	A temperature at which equilibrium vapour pressure of a liquid is equal to the total pressure on the surface is _________.

5.	Ratio of the reacting amount of a component to its initial amount is _________.

6.	The law of conservation of mass is outlined as Input - _________= Output.

7.	Steady state refers to _________.

8.	Recycle ratio is defined as _________.

9.	Number of times a reaction rate changes with a 100C change in temperature is _________.

10.	In _________ reaction, the substrate is referred as reactant.

PART – B (5 x 3 = 15 MARKS)

11.	Prove the dimensional consistency for the following

a.	q = h A T		b.	1 Pa.s = 1 x 103 CP

12.	Explain how the bubble point for a solution containing two components is computed. The system obeys Raoult`s law and behaves like an ideal solution and like an ideal gas.

13.	 A food material containing 80% moisture content (wet basis) is being dried to 5% (wet basis) using air. The inlet humidity of the air is 0.002 and its exit humidity is 0.02. Calculate the amount of dry air needed.

14.	Pure ethylene is heated from 30oC to 250oC at a constant pressure. Calculate the heat added per k mole.

15.	At 500 K the rate of a reaction is ten times the rate at 400 K. Find the activation energy for the reaction using Arrhenius law.

PART – C (5 x 15 = 75 MARKS)

16.	The solution of NaCl in water contains 203 g of NaCl per litre at 200C. The density of the solution at this temperature is 1.148 g/ml. Find the composition in

a.	weight % 	b.	volume % of water	c.	mole %		d.	atomic %	e.	molality

f.	g NaCl/g water.

(OR)

17.	The average molecular weight of a flue gas sample is calculated by two different engineers. One person used the correct molecular weight of N2 as 28, while the other used an incorrect value of 14. They got the average molecular weight as 30.08 and the incorrect one as 18.74. Calculate the % volume of N2 in the flue gases. If the remaining gases are CO2 and O2. Calculate their composition also.

[P.T.O]

18.	a.	Calculate the weight of NaCl that should be placed in a 1 liter volumetric flask to prepare a 	solution of 1.8 molality. Density of this solution is 1.06 g/cc.

b.	A mixture of NH3 and air at 730 mm Hg and 30C contains 5.1% NH3 by volume. This gas is passed at a rate of 100 ft3/min. through an absorption tower in which NH3 is removed. The gases leave the tower at 725 mm Hg and 20OC having 0.05% NH3 by volume. Calculate rate of flow of gases leaving the tower	ii) weight of NH3 absorbed

(OR)

19.	a.	Write the Vander Waal`s equation.

b.	A gas mixture used for controlled atmosphere storage of vegetables contains 5% CO2, 5% O2 and 90% N2. The mixture is generated by mixing appropriate quantities of air and N2 and CO2 gases. Of this mixture, 100 m3 at 200C and 1 atm is needed per hour. Air contains 21% O2 and 79% N2. Calculate the rate at which the component gases must be metered into the system in m3/ hr at 200C and 1 atm.

20.	Carbon tetrachloride is made as follows:

CS2   +   3Cl2 		→    CCl4    +    S2Cl2

212.76	        153.84

The product gases are found to contain CCl4 33.3 %; S2Cl2 33.3%; CS2 1.4% and Cl2 32%. Calculate:	a.	percentage of the excess reactants used	b.	percentage conversion

c.	kg of CCl4 produced per 100 kg Cl2 converted.

(OR)

21.	In order to obtain barium in a form that may be put into solution, the natural sulphate (barites) containing only pure barium sulphate and infusible matter is fused with an excess of pure anhydrous soda ash. Upon analysis of the fusion mass it is found to contain 11.3% barium sulphate, 27.7% sodium sulphate and 20.35% sodium carbonate. The remainder is barium carbonate and infusible matter. Calculate the following

a.	% conversion of barium sulphate to the carbonate

b.	Composition of the original barites

c.	% excess of the sodium carbonate used in excess of the theoretically needed amount for all 	the barium sulphate

22.	a.	Define Sensible heat

b.	Calculate the amount of water that must be supplied to heat exchanger that cools 100 kg/hr of tomato paste from 900 to 200C. The tomato paste contains 40% solids. The increase in water temperature should not exceed 100C while passing through the heat exchanger. There is no mixing of water and tomato paste in the heat exchanger.

(OR)

23.	a.	Define Latent heat.

b.	Calculate the amount of steam at 121.10C that must be added to 100 kg of a food product with a specific heat of 3559 J/kg. K to heat the product from 4.440C to 82.20C by direct steam injection.

24.	Explain the various theories of reaction rates with suitable example.

(OR)

25.	Write short notes on:

a.	Temperature dependence of reaction rates     	b.	Enzymatic reaction