# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	FUNDAMENTALS OF THERMAL FLUID SCIENCES FOR FOOD

# ENGINEERS 								      	   		       Time: 3 hours

# Subject Code:	CL215		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	_______ is a phenomenon of rise or fall of liquid surface relative to the adjacent general level of liquid.

2.	_______ states that the shear stress on fluid element layer is directly proportional to the rate of shear strain.

3.	A system is said to be in thermodynamic equilibrium if its _______, _______ and_______ remain unchanged.

4.	_______ system in which mass as well as energy cannot be exchanged with the surroundings.

5.	The driving force of heat transfer is _______.

6.	A material which retards the flow of heat with reasonable effectiveness is known as _______.

7.	_______ fluid has no viscosity and surface tension and is incompressible.

8.	The rate of emission of radiation by a body depends upon _______ and _______.

9.	In stable film boiling, the heat transfer is due to both _______and _______.

10.	The phase change of solid to vapour state is _______.

PART – B (5 x 3 = 15 MARKS)

11.	Define Reynolds number and the range for various flows.

12.	Differentiate between open and closed system.

13.	The inner surface of a plane brick wall is at 60C and the outer surface is at 35C. Calculate the rate of heat transfer per square meter of surface area of the wall, which is 220 mm thick. The thermal conductivity of the brick is 0.51 W/moC.

14.	What are the characteristics of a boundary layer?

15.	Explain the boiling heat transfer phenomena.

PART – C (5 x 15 = 75 MARKS)

16.	Derive the Bernoulli equation of motion with their assumptions.

(OR)

17.	An orifice meter with orifice diameter 15cm is inserted in a pipe of 30 cm diameter the pressure on the upstream and downstream of orifice meter is 14.7 N/ cm2 and 9.81N/cm2 respectively. Find the discharge.  Take, co-efficient of discharge as 0.6.

18.	Nitrogen gas is confined in a cylinder and the pressure of the gas is maintained by a weight placed on the piston. The mass of the piston and the weight together is 50 kg. The acceleration due to gravity is 9.81 m/s2 and the atmospheric pressure is 1.01325 bar. Assume frictionless piston.   Determine (a) Force exerted by the atmosphere, the piston, and the weight on the gas if the piston is 100 mm in diameter (b) Pressure of the gas (c) If the gas is allowed to expand pushing up the piston and the weight by 400 mm, what is the work done by the gas in kJ? (d) What is the change in the potential energy of the piston and the weight after the expansion?

(OR)

[P.T.O]

19.	Write short notes on:

a.	First law of thermodynamics		b.	Second law of thermodynamics

c.	Zeroth law of thermodynamics 	d.	Thermodynamics properties

e.	Thermodynamic process

20.	A cold storage room has walls made of 220 mm of brick on the outside, 90 mm of plastics foam, and finally 16 mm of wood on the inside. The outside and inside air temperatures are 25oC and -3oC respectively. If the inside and outside heat transfer coefficients are respectively 30 and 11 W/m2 oC, and the thermal conductivities of brick, foam and wood are 0.99, 0.022 and 0.17 W/m oC respectively, determine: (i) Rate of heat removed by refrigeration if the total wall area is 85m2  (ii) Temperature of the inside surface of the brick.

(OR)

21.	A small electric heating application uses wire of 2 mm diameter with 0.8 mm thick insulation (k = 0.12 W/m0C). The heat transfer coefficient (ho) on the insulated surface is 35 W/m2 oC. Determine the critical thickness of insulation in this case and the % change in the heat transfer rate if the critical thickness is used, assuming the temperature difference between the surfaces of the wire and surrounding air remains unchanged.

22.	a.	Explain how dimensional analysis is used in convection phenomenon where heat transfer 	coefficient is found to be dependent on diameter, density, velocity, specific heat, thermal 	conductivity and viscosity.														(10)

b.	Discuss the heat transfer coefficient for laminar and turbulent flow inside a pipe. 	(5)

(OR)

23.	Explain about:																      (3 x 5)

a.	Black body concept 		b.	Plank`s law		c.	Kirchhoff`s law

24.	Briefly explain the pool boiling curve for water with neat diagram.

(OR)

25.	Dry saturated steam at a pressure of 2.45 bar condenses on the surface of a vertical tube of height 1 m. The tube surface temperature is kept at 1170C. Estimate the thickness of the condensate film and the local heat transfer co-efficient at a distance of 0.2 m from the upper end of the tube.