# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	MECHANICS OF FLUIDS 								       Time: 3 hours

# Subject Code:	09CE202	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Write the relationship between dynamic viscosity and kinematic viscosity.

2.	Define ‘center of pressure’.

3.	Write the continuity equation for three dimensional flow.

4.	What is the stream line?

5.	Name the equipments used to measure the discharge in a pipeline.

6.	List the different types of weirs.

7.	What are the types of minor losses?

8.	What is the maximum efficiency of power transmission through pipes?

9.	List the fundamental dimensions used in the dimensional analysis.

10.	What is the general classification of models?

PART – B (5 x 3 = 15 MARKS)

11.	Define ‘Surface tension’.

12.	What are the assumptions made for Bernoulli’s equation?

13.	Differentiate venturimeter and orifice meter.

14.	What are the equivalent pipes?

15.	Define Buckingham Pi theorem.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Determine the specific gravity of a fluid having viscosity 0.05poises and kinematic 	viscosity 0.035 stokes.															(5)

b.	A circular opening, 3m diameter, in a vertical side of a tank is closed by a disc of 3m diameter which can rotate about a horizontal diameter. Calculate: (i) the force on the disc and (ii) the torque required to maintain the disc in equilibrium in the vertical position when the head of water above the horizontal diameter is 4m.											(10)

(OR)

17.	a.	Find the surface tension in a soap bubble of 40mm diameter when the inside pressure is 2.5 	N/m2 above atmospheric pressure.													(5)

b.	A 15cm diameter vertical cylinder rotates concentrically inside another cylinder of diameter 15.1cm. Both cylinders are 25cm high. The space between the cylinders is filled with a liquid whose viscosity is unknown. If a torque of 12 Nm is required to rotate the inner cylinder at 100rpm, determine the viscosity of the fluid.											(10)

18.	a.	Derive the continuity equation for three dimensional flow.								(7)

b.	If for a two dimensional potential flow, the velocity is given by Φ = x(2y-1). Determine the velocity at the point P(4,5). Determine also the value of the stream function at the point P.(8)

(OR)

[P.T.O]

19.	a.	Derive the Bernoulli’s equation for a streamline flow.									(10)

b.	A 30cm diameter pipe, conveying water, branches in to two pipes of diameters 20cm and 15cm respectively. If the average velocity in the 30cm diameter pipe is 2.5 m/s, find the discharge in this pipe. Also determine the velocity in 15cm pipe if the average velocity in 20cm diameter pipe is 2 m/s.														(5)

20.	Derive the Hagen – Poiseuille equation for the viscous flow.

(OR)

21.	a.	A fluid of viscosity 0.7Ns/m2 and specific gravity 1.3 is flowing through a circular pipe of 	diameter 100mm. the maximum shear stress at the pipe wall is given as 196.2N/m2, find 	(i) 	the pressure gradient, (ii) the average velocity and (iii) Reynolds number of the flow.		(9)

b.	A rectangular channel 2m wide has a discharge of 250 litres per second, which is measured by a right angled triangular notch weir. Find the position of the apex of the notch from the bed of the channel if maximum depth of water is not to exceed 1.3m. Take coefficient of discharge as 0.62.																			(6)

22.	For a town water supply, a main pipe line of diameter 0.4m is required. As pipes more than 0.35m diameter are not readily available, two parallel pipes of the same diameter were used for water supply. If the total discharge in the parallel pipes is same as in the single main pipe, find the diameter of the parallel pipe. Assume the coefficient of friction same for all pipes.

(OR)

23.	a.	Find the loss of head when a pipe of diameter 200mm is suddenly enlarged to a diameter of 	400mm. the rate of flow of water through the pipe is 0.25m3/s. 						(6)

b.	A pipe of diameter 300mm and length 3500m is used for the transmission of power by water. The total head at the inlet of the pipe is 500m. Find the maximum power available at the outlet of the pipe, if the value of f = 0.006.											(9)

24.	The efficiency η of a fan depends on density ρ, dynamic viscosity µ of the fluid, angular velocity ω, diameter D of the rotor and the discharge Q. Express η in terms of dimensionless parameters by the Buckingham Pi theorem method.

(OR)

25.	a.	A ship 300m long moves in sea water, whose density is 1030kg/m3.  A 1:100 model of this 	ship is to be tested in a wind tunnel. The velocity of air in the wind tunnel around the 	model is 30m/s and the resistance of the model is 60N. Determine the velocity of ship in 	sea 	water and also the resistance of the ship in sea water. The density of air is given as 	1.24kg/m3. Take the kinematic viscosity of sea water and air as 0.012 stokes and 0.018 	stokes respectively. 																(8)

b.	In the model test of a spillway the discharge and velocity of flow over the model were 2m3/s and 1.5m/s respectively. Calculate the velocity and discharge over the prototype which is 36 times the model size.																(7)