# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	MECHANICS OF DEFORMABLE BODIES  - I 			       Time: 3 hours

# Subject Code:	09CE205	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define Modulus of Rigidity.

2.	What is the stress due to suddenly applied load?

3.	Define Principal planes.

4.	What is the use of Mohr`s Circle?

5.	What is the relationship between rate of loading and shear force in a beam?

6.	Define point of contraflexure.

7.	What is the section modulus of a beam of rectangular section?

8.	What is the strain energy due to bending moment?

9.	Define stiffness of spring.

10.	What is the power developed by a shaft rotating at a speed of N r.p.m. and subjected to a torque of T Nm?

PART – B (5 x 3 = 15 MARKS)

11.	Define Yield stress, Ultimate stress and Breaking stress.

12.	What is the hoop stress and longitudinal stress in a thin cylindrical shell of diameter ‘d’, thickness ‘t’, Poisson`s ratio ‘1/m’ , Modulus of Elasticity of the material ‘E’ subjected to internal pressure ‘p’?

13.	What are the various types of beams?

14.	Draw a neat sketch showing the variation of shear stress in I-section.

15.	State the assumptions in the theory of pure torsion.

PART – C (5 x 15 = 75 MARKS)

16.	 The following data relate to a bar subjected to a tension test in Universal Testing Machine: Diameter of the bar, d=30mm

Tensile load,   =54kN

Gauge length =300mm

Extension of the bar =0.112mm

Change in diameter = 0.00366mm

Determine i) Poisson`s ratio ii) The values of three modulii.

(OR)

17.	Derive the relationship between Young`s Modulus ‘E’ and   Bulk Modulus ‘K’.

18.	How do you determine the Principal stresses and maximum shear stress graphically?

(OR)

[P.T.O]

19.	A direct stress of 120 MN/m2 in tension and 90 MN/m2 in compression are applied to an elastic material at a certain point on planes at right angles to another. If the maximum principal stress is not to exceed 150 MN/m2 in tension, to what shearing stress can the material be subjected? What is the maximum shearing stress in the material as a result and also determine the magnitude of minor principal stress.

20.	Draw the BM and SF diagrams for a cantilever loaded as shown in figure.

(OR)

21.	Draw the BM and SF diagrams for the beam shown in figure.

22.	A leaf spring of semi-elliptical type has 11 plates each 9cm wide and 1.5cm thick. The length of spring is 1.5m. The plates are made of steel having a proof stress of 600 MN/m2. To what radius should the plate be bent initially?

From what height can a load of 600N fall on to the centre of the spring, if maximum stress is to 	be one-half of the proof stress?

(OR)

23.	A 250 mm x150mm rectangular beam is subjected to maximum bending moment of 750 kNm. Determine:

a.	The maximum stress in the beam.

b.	If the value of E for the beam material is 200GN/m2, find out the radius of curvature for 	that portion of the beam where the bending is maximum.

c.	The value of the longitudinal stress at a distance of 65mm from the top surface of the beam.

24.	A close coil helical spring has a stiffness of 10 N/mm. Its length when fully compressed with adjacent coils touching each other is 400mm. The modulus of rigidity of material of the spring C =  8 x 10 4 N/mm2.

a.	Determine the wire diameter and mean coil diameter, if their ratio= 1/10.

b.	If the gap between any two adjacent coils is 2mm, what maximum load can be applied 	before the spring becomes solid

c.	What is the corresponding maximum shear stress in the spring?

(OR)

25.	A circular bar made of cast iron is to resist an occasional torque of 2.2kNm acting in transverse plane. If the allowable stresses in compression, tension, and shear are 100MN/m2, 35 MN/m2 and 50 MN/m2 respectively, find:

a.	Diameter of the bar

b.	Angle of twist under the applied torque per metre length of the bar.  Take C as 40 GN/m2.