# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	MECHANICS OF DEFORMABLE BODIES II 			       Time: 3 hours

# Subject Code:	09CE206	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define slope of a point in a deflected beam.

2.	A beam is simply supported with span ’l’, an eccentric load ‘w’ is acting at a distance ‘a’ from one support and ‘b’ from another support. Write the expression for finding the deflection under the load.

3.	Define slenderness ratio.

4.	Write the expression that states the limitations for Euler’s formula for columns.

5.	Maximum principal strain theory is also called as __________.

6.	State the maximum shear stress theory.

7.	Write the expression for finding the factor ‘h2’ or ‘m’ in curved rectangular beams.

8.	Write any two assumptions made in the derivation of stresses in a curved bar.

9.	Define centroidal principal axes of a section in unsymmetrical bending.

10.	Write the expression for finding Iuu in a beam subjected to unsymmetrical bending with given Ixx, Iyy and Ixy.

PART – B (5 x 3 = 15 MARKS)

11.	Differentiate conjugate beam from original loaded beam.

12.	Write the classification of columns according to slenderness ratio. Also write the limiting slenderness ratio for each type.

13.	Write the straight line formula in column analysis.

14.	Write the assumptions made while deriving the relation known as Winkler Boch formula for finding stress in curved beams.

15.	A wooden beam of cross section 100 mm x 150 mm is used to support a sloping Mangalore tiled roof

Muu = 5.196 kNm.		Iuu = 28125000 mm4

Mvv = 3 kNm.			Ivv = 12500000 mm4

Find the maximum tensile stress in the beam.

PART – C (5 x 15 = 75 MARKS)

16.	A simply supported beam AB, supported at A and B is loaded with two point loads 30 kN each at a distance of 2 m and 3 m respectively from the end A. Determine the position and magnitude of maximum deflection by Macaulay’s method. E = 2 x 105 N/mm2 I = 7200 x 104mm4

(OR)

[P.T.O]

17.	For the beam shown in the figure, determine the following:

a.	Slope at A 	b.	Deflection at the mid span I = 8 x 10-5 m4     	E = 200 x 106 kN/m2

15kN

5m

B

D

A

I

4I

10m

8m

C

18.	A hollow mild steel tube 40 mm internal diameter and 5 mm thick is used as a strut with both ends hinged. Take length of the column as 6m. Find the crippling load and safe load taking factor of safety as 3. E = 2 x 105 N/mm2

(OR)

19.	The external and internal diameter of a hollow cast iron column 50 mm and 40 mm respectively. If the length of this column is 3 m and both of its ends are fixed, determine the crippling load using Rankine’s formula. c = 550 N/mm2    a = 1/1600.

20.	Find the thickness of metal necessary for a cylindrical shell of internal diameter 160 mm to withstand an internal pressure of 8 N/mm2. The maximum hoop stress in the section is not to exceed 35 N/mm2.

(OR)

21.	Determine the diameter of a bolt which is subjected to an axial pull of 9kN together with a transverse shear force of 4.5kN using maximum principal stress theory. Given the elastic limit in tension is 225 N/mm2. Factor of safety = 3, Poisson’s Ratio = 0.3

22.	Find the position of shear center for a channel section placed with the flanges horizontal. The overall depth is 250mm with flange outer dimension is 105 mm. The thickness of the section is 10 mm throughout. A shear force of 60kN acts through the shear center. Neglect the minor shear force. Draw the shear flow for the channel section.

(OR)

23.	Determine the:	a.	location of neutral axis	 b.	maximum and minimum stress

c.	ratio of maximum and minimum stress.

when a curved beam of rectangular cross section of width 20 mm and of depth 40 mm is subjected to pure bending moment +600 Nm. The beam is curved in a plane parallel to the depth. The mean radius of curvature is 50 mm.

24.	Determine the centroidal principal moment of inertia of the equal angle section 30 mm x 30 mm x 10 mm.

(OR)

25.	An Indian Standard ‘I’ section ISMD 400 has the following properties

Depth of I section = 400 mm

Width of I section = 140 mm

Ixx = 204.584 x 106 mm4

Iyy = 6.221 x 106 mm4

Determine the percentage increase in stress at right hand top corner of the section if the plane of loading is 5 to vertical compared to exactly vertical loading.