# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	THEORIES AND PRACTICES OF WATER AND WASTEWATER

# ENGINEERING 								      	   	       Time: 3 hours

# Subject Code:	09CE211	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	As per IS 1172 - 1993, water supply for domestic purpose to be provided for a person is _________ litres per day.  Out of which, requirement for consumption for drinking is _________ litres.

2.	The chemical analysis of disinfected water, which can confirm the absence of bacteria, is_________.

3.	Define “sullage”.

4.	Name any two tests which can give organic strength of sewage.

5.	Name the pipe material, which is highly durable and also corrosion resistive.

6.	What are pipe appurtenances?

7.	Name the distribution network layout that requires least cost.

8.	What is equivalent pipe?

9.	Limiting velocities of flow in a sewer are _________ and _________.

10.	What are sanitary fittings?

PART – B (5 x 3 = 15 MARKS)

11.	What is design period? List the factors influencing it.

12.	State the sanitary significance of BOD/COD ratio.

13.	Define economical diameter of pumping main.

14.	Compare the continuous and intermittent types of water supply.

15.	What are drop manholes?  Brief their formation.

PART – C (5 x 15 = 75 MARKS)

16.	a.	The following is the population data of a city, available from census records. Determine 	the population of the city in 2021 by Logistic curve method.						(8)

| Year | 1931 | 1941 | 1651 | 1961 | 1971 | 1981 | 1991 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Population | 12000 | 16500 | 26800 | 41500 | 57500 | 68000 | 74100 |

b.	Describe the formation and designing of infiltration galleries.						(7)

(OR)

17.	a.	Explain the membrane filter technique. 											(8)

b.	Write a note on water demand fluctuations and their significance in water supply scheme 	design. 																		(7)

18.	a.	Explain BOD test and its limitation. 											(8)

b.	The 5 day BOD at 30C for a sewage sample is 6540 mg/l. Calculate the ultimate BOD of 	sewage and its population equivalent, if the sewage flow rate is 5 MLD.				(7)

(OR)

[P.T.O]

19.	a.	Explain the method of estimating storm water flow for the designing of storm sewers.(8)

b.	What is relative stability of sewage?  Describe its estimation. 						(7)

20.	The lowest level of an impounding reservoir situated in hills and being used as a source of water supply for a town, 170 m higher than the level of the service reservoir and the distance between them is 60 km.  The storage reservoir and the service reservoir were initially connected by a single pipeline designed to carry a maximum daily demand of 25 million litres per day.  Later it was found necessary to increase the supply to 36 million litres per day.  It was therefore decided to lay another pipe of same diameter parallel to the first one for a certain required length and cross connected.  Calculate the diameter and the length of the second pipe line required to be laid.  Use Hazen-Williams formula with CH = 120.

(OR)

21.	a.	It is required to increase in a section of an existing water supply scheme the effective 	pressure by 6 m and the discharge by 40 percent.  The distribution main is 600 mm 	diameter and the length leading to the section is 600 m and the present discharge is 13650 	litres per minute. Design a suitable booster. Take friction factor for pipe as 0.03 and 	efficiency of motor-pump unit as 0.8. 											(8)

b.	What are advantages and disadvantages of cement concrete pipes?  Describe the method of joining of concrete pipes. 														(7)

22.	Calculate the storage capacity of service reservoir to supply for meeting the demand given below if the inflow of water to the service reservoir is maintained at a uniform rate throughout the period of pumping for 8 hours during 06 – 10 and 16 – 20 hours

| Time (hours) | 00 - 04 | 04 - 08 | 08 - 10 | 08 - 12 | 12 - 16 | 16 - 20 | 20 - 24 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Demand (Million litres) | 0.18 | 0.67 | 1.23 | 0.35 | 0.60 | 1.22 | 0.34 |

(OR)

23.	a.	Explain the Hardy - Cross method of analysing pipe network by balancing heads by 	correcting assumed flow.  Derive the expression for the same. 						(8)

b.	Describe the service connection with a neat sketch showing its components. 		(7)

24.	a.	A 500 mm diameter sewer is to discharge 0.1 cumecs at a velocity, as self-cleansing as a 	sewer running full at 0.9 m/sec. Find the depth and velocity of flow and the required slope. 	Take uniform value of N = 0.013.

(OR)

25.	Explain the laying of sewer for the desired alignment and gradient. Also discuss the different tests carried out on a laid sewer.