# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	GEOTECHNICAL ENGINEERING						       Time: 3 hours

# Subject Code:	09CE212						       				       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is an alluvial soil?

2.	Write down the value of unit weight of water in terms of gram per cc.

3.	What is coarse grained soil?

4.	What is Plasticity Index?

5.	Define Darcy’s Law.

6.	Define Seepage.

7.	What do you mean by Consolidation?

8.	Write down the equation for the determination of Coefficient of compressibility.

9.	Define Cohesion.

10.	Write down the Taylor’s Stability Number Equation.

PART – B (5 x 3 = 15 MARKS)

11.	Define Void ratio and Porosity.

12.	What are the different Consistency Limits?

13.	Define effective and pore water pressure.

14.	What are the components of settlement?

15.	How do you classify the shear strength according to drainage conditions?

PART – C (5 x 15 = 75 MARKS)

16.	Determine the (i) Water Content, (ii) Dry density, (iii) Bulk density, (iv) Void ratio and (v) Degree of saturation from the following data:

Sample size 3.81 cm dia.  X 7.62 cm ht.

Wet weight                  = 1.668 N

Oven-dry weight         = 1.400N

Specific gravity           = 2.7

(OR)

17.	Explain the test procedure for the determination of Specific gravity and Water content.

18.	Explain various grain size classification of soils.

(OR)

19.	Define Liquid Limit, Plastic Limit and Shrinkage Limit. Explain in detail laboratory tests for the determination of Liquid Limit.

20.	Explain the laboratory determination of permeability and derive the equation for Variable Head Method.

(OR)

[P.T.O]

21.	Water flows at the rate of 0.09 ml/s in an upward direction through a sand sample with a coefficient of permeability of 2.7 x 10-2 mm/s.  The thickness of the sample is 120 mm and the area of cross-section is 5400 mm2.  Taking the saturated unit weight of the sand as 18.9 kN/m3, determine the effective pressure at the middle and bottom of the sample.

22.	The soil profile at a building site consists of dense sand up to a 2 m depth, normally loaded soft clay from 2m to 6m depth, and stiff impervious rock below 6 m depth. The ground- water table is at 0.4 m depth below ground level. The sand has a density of 18.5 kN/m3 above water table and 19 kN/m3 below it. For the clay, natural water content is 50%, Liquid limit is 65% and grain specific gravity is 2.65.  Calculate the probable ultimate settlement resulting from a uniformly distributed surface load of 40 kN/m2 applied over an extensive area of the site.

(OR)

23.	A saturated soil has a compression index of 0.25.Its void ratio at a stress of 10kN/m2 is 2.02 and its permeability is 3.4 x 10-7mm/s.  Compute:

a.	Change in void ratio if the stress is increased to 19 kN/m2;

b.	Settlement in (a) if the soil stratum is 5m thick; and

c.	Time required for 40 % consolidation if drainage is one – way.

24.	Explain Friction Circle Method of stability analysis.

(OR)

25.	The following data relate to a triaxial compression tests performed on a soil sample:

| Test No. | Chamber Pressure | Max.deviator stress | Pore pressure at max. deviator stress |
| --- | --- | --- | --- |
| 1 2 3 | 80 kN/m2 150 kN/m2 210 kN/m2 | 175 kN/m2 240 kN/m2 300 kN/m2 | 45 kN/m2 50 kN/m2 60 kN/m2 |

Determine the total and effective stress parameters of the soil.