# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	DESIGN OF MASONRY AND TIMBER STRUCTURES 	       Time: 3 hours

# Subject Code:	09CE213	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What do you understand by ‘strength’ of a structure?

2.	Define ‘stiffness’ of a material.

3.	What is the purpose of shear wall?

4.	Name the factors that affect the strength of the masonry structure.

5.	What is the use of bed block?

6.	What is middle third rule?

7.	Give an example of free standing wall.

8.	What are the types of bands?

9.	Classify solid columns.

10.	Name the permissible defects in timber.

PART – B (5 x 3 = 15 MARKS)

11.	Give the classification of structures based on function, material and shape.

12.	What are the various types of masonry walls?

13.	What are the conditions for the stability of masonry structures on soil?

14.	Bring out any two points while designing the earthquake resistant structures.

15.	Differentiate solid and built up columns.

PART – C (5 x 15 = 75 MARKS)

16.	Summarize the working stress and limit state methods of design.

(OR)

17.	Bring out the details of how wind load and seismic load for a structure is calculated.

18.	Design the masonry column of square cross section which is subjected to an axial force of 400 KN with an eccentricity of 60mm. Take the height of the column as 4m. Assume that the column is hinged at both the ends.  Assume the strength of brick masonry as 12.5 N/mm2.

(OR)

19.	Determine the allowable axial load per metre length of a 200mm solid wall. The height of the wall between plinth wall and slab is 3.5m. The length between the cross wall is 5m and it is continuous. Take the compressive stress for the wall as 0.55N/mm².

20.	A masonry trapezoidal dam 4m high, 1m wide at its top and 3m wide at its bottom retains water on its vertical face. Determine the maximum and minimum stress at the base at the following conditions

a.	When the reservoir is full		b.	When the reservoir is empty.

Take the weight of the masonry as 20kN/m³                                         			      (7.5 + 7.5)

(OR)

[P.T.O]

21.	A column section 400mm x 800mm carries a load of 100 kN acting at 160mm from the 800mm face and 350mm from the 400mm face as shown in the figure. Determine the stress intensities at the four corners of the column.

160mm

•

w

400mm

350mm

800mm

22.	Bring out a brief note on the recommendations for the various structural elements of masonry building to resist earthquake.

(OR)

23.	Check the vertical bending of the wall for a room of dimension 5m x8m against earthquake. The walls are built by 0.2m brick wall with 1:6 cement and sand mortar. The height of the building from plinth level to roof level is 3.5m and the height of the wall is 1.4m above the lintel. The dead load and live load of the roof are 3000N/m² and 5000N/m². The earthquake coefficient can be taken as 0.16.

24.	A timber beam having a clear span of 6 meters carries a uniformly distributed load of 15 KN/m including the self weight of the beam. Assume the beam is to be made of deodar wood, design the beam. Apply the check for deflection and bearing.  E=12.7 kN/mm2.  Assume inside location in the design.

(OR)

25.	Design a solid deodar wood column to the following requirements.

The load on the column – 450KN

The location of the column – outside

Effective length of the column -3m

Take the safe compressive stress as 7N/mm².