# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	REINFORCED CONCRETE STRUCTURES – II   		       Time: 3 hours

# Subject Code:	09CE217	       	               	       	  			              Maximum Marks: 100

Use of IS 456-2000, IRC Codes, IS 3370, IS 1893-2002 and Design Aids (SP16) permitted.

Use M20 concrete and Fe415 steel unless otherwise specified.

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is the use of retaining wall?

2.	If tread and rise of a stair are 250 mm and 150 mm, respectively, determine the slope of the flight with respect to horizontal.

3.	What is hoop stress?

4.	What is staging in a water tank?

5.	List the commonly adopted live load classes while designing a road bridge.

6.	What is the need for providing cross-girders in T-beam slab-bridge?

7.	What is a building frame?

8.	State any two approximate methods used for lateral load analysis of multi-storey frames.

9.	For complete yield line pattern to develop, the slab must be _________ reinforced.

10.	Draw the yield line pattern for a simply supported circular slab.

PART – B (5 x 3 = 15 MARKS)

11.	Draw a typical cross section of a cantilever retaining wall and indicate the location of tension reinforcement in all its components.

12.	Draw a neat sketch of INTZE type water tank and locate its parts.

13.	Write the details of class-A loading on bridges.

14.	What is a substitute frame? Give an example.

15.	Write down the characteristic features of yield line.

PART – C (5 x 15 = 75 MARKS)

16.	Design one of the flights of an ‘open newel stair’ with quarter space landing using the following data.  No. of steps in the main flight = 9.  Tread T = 250 mm, Rise R = 150 mm.  Width of landing and flight = 1.50 m.  Width of the wall is = 400 mm.  Staircase dimensions = 5.25 X 4.25 m.  Use M 20 concrete and steel grade Fe 415 HYSD bars.  Assume a bearing of 200 mm in the walls.  Assume live load = 3000 N/mm2.   Use design constant: Rc = 2.761.   Find: Reactions, Mmax, d, and Ast.  Sketch the details of reinforcement neatly.

(OR)

17.	Design a T-shaped cantilever retaining wall to retain earth embankment 3 m high above ground level using the following data. The unit weight of earth is 18 kN/m3 and its angle of repose  is 30.   The embankment is horizontal at its top.  The safe bearing capacity (SBC) may be taken as 100 kN/m2, and the coefficient of friction between soil and concrete as 0.5, and use M 20 concrete and steel grade Fe 415 HYSD bars.  Use design constant Ru = 2.761.  Assume the depth of foundation as 1 m.  Find dimensions of the base, thickness of stem, stability of wall (factor of safety) against overturning, design the stem (i.e. d, Ast), and find the distribution and temperature steel.

[P.T.O]

18.	Design the following components of a circular tank with fixed base for a capacity of 400,000 litres with data given below.  The depth of water is to be 4m including the free board of 250 mm. Use M 20 concrete and grade I mild steel reinforcement.  Assume μ = 0.  The tank is free at the top and rests on ground.  Take unit weight of water as 9.8 kN/m3.

a.	Determine the dimensions of the tank.

b.	Describe the procedure to determine the hoop tension and bending moment by any one method.

c.	Design of sidewall for max BM, if max BM is 9083 N-m at the base.

(OR)

19.	A rectangular water tank 4.5 m long, 2.25 m wide and 2.25 m high has its walls rigidly jointed at the vertical edges and pin jointed at their horizontal edges.  Design the tank if it is supported on all sides under the wall.  Use M 20 concrete and mild steel reinforcement bars.

a.	Determine the moment and shear

b.	Design the thickness of tank walls

c.	Determine the reinforcement in the horizontal direction

d.	Determine the reinforcement in the vertical direction

e.	Sketch the details of reinforcement neatly.

20.	Design a solid slab bridge for class A loading for the following requirements:  Clear span = 4.5 m.  Clear width of carriageway = 7.0 m.  Average thickness of wearing coat = 80 mm.  Use M 20 concrete and mild steel grade I bars (Use: permissible stresses in steel σst 125 N/m2). Take unit weight of concrete as 24 kN/m3.  Width of kerb = 0.5 m. Assume overall thickness of slab as 330 mm with effective depth as 300 mm.  Assume K = 2.48 corresponding to L/l = 1.67.  Design for bending only.

(OR)

21.	What is Courbon’s method? Explain the method with neat sketch and its applications in detail. Specify the conditions under which Courbon’s method of bridge design is suitable.

22.	Using portal method analyze the building frame in the Figure to determine the axial force and bending moment in the column AB at A.

3 m

3 m

3.8 m

3 m

20 kN

30 kN

A

B

(OR)

23.	Explain the analysis for horizontal loads in building frames by approximate methods:

a.	Portal Method, 	b.	Cantilever Method.

24.	Derive an expression for the moment of an isotropically reinforced square slab fixed on all edges and subjected to UDL.

(OR)

25.	A square slab of side length 5 m is simply supported along all the four ends and carries a service load of 3 kN/m2.  . Design the slab, using virtual load method.  Use M 15 concrete and mild steel bars.