# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	STRUCTURAL ANALYSIS-II    	   						       Time: 3 hours

# Subject Code:	09CE218	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is a fixed end moment?

2.	What do you mean by statically indeterminate beam?

3.	What is carry-over moment?

4.	Write the fixed end moments for a fixed beam with triangular load of load intensity “w” at one end and zero at other end.

5.	Define: Degree of freedom.

6.	What is “lack of fit”?

7.	State any one difference between stiffness and flexibility method.

8.	What is Static Condensation?

9.	What is damping?

10.	What is free vibration?

PART – B (5 x 3 = 15 MARKS)

11.	Write the steps to solve any structurally indeterminate frame by slope-deflection method.

12.	Explain the concept of distribution factor in moment-distribution method with an example.

13.	Will there be any difference in applying flexibility method for determinate structures and indeterminate structures? Justify your answer.

14.	Explain the concept of Kinematic Indeterminacy.

15.	Bring out the difference between damped and undamped vibrations.

PART – C (5 x 15 = 75 MARKS)

16.	A continuous beam is supported and loaded as shown in figure. Analyse the beam for support moments and reactions. Take E = 200 x 106 kN/m2 and I=100x106 mm4 constant throughout. Use Slope-Deflection method.

(OR)

17.	Analyse the frame shown in figure for moments at the ends of members. EI is constant for all the members.

[P.T.O]

18.	Analyse the continuous beam shown in figure by Moment Distribution method. Sketch the bending moment and shear force diagrams.

(OR)

19.	Analyse the portal frame loaded as shown in figure by the Moment Distribution method and sketch the bending moment and shear force diagram.

20.	Analyse the continuous beam in figure by flexibility method. Draw the BMD.

(OR)

21.	Develop the flexibility matrix for the beam shown in figure.

[P.T.O]

22.	Using matrix stiffness method, analyse the truss for the member forces in the truss loaded as shown in figure. Length AD = 400 mm, BD = 461.9 mm, CD = 800 mm. Take AE/l = 1 for all the members.

(OR)

23.	Analyse the frame shown in figure by Matrix Stiffness method. Draw the BMD.

24.	A vibrating system consists of a mass 5 kg, spring of stiffness 120 N/m and a damper with a damping coefficient of 5 N-s/m. Determine:

a.	The damping factor			b.	Natural frequency of damped vibration

c.	Logarithmic decrement		d.	The ratio of two successive amplitudes

The number of cycles after which the initial amplitude is reduced to 25%.

(OR)

25.	Derive the equations of motion for undamped SDOF system.