# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	FLUID MECHANICS AND MACHINERY     			       Time: 3 hours

# Subject Code:	09CE242	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What do you mean by the term ‘viscosity’?

2.	Define Atmospheric pressure and Absolute pressure.

3.	What is a flow-net?

4.	Write the Cauchy Riemann equation.

5.	What is velocity of approach?

6.	What is an equivalent pipe?

7.	What is use of air vessel?

8.	What do you understand by ‘Indicator Diagram’?

9.	Where is Kaplan turbine used?

10.	What is a surge tank?

PART – B (5 x 3 = 15 MARKS)

11.	Derive the expression for height of capillary rise.

12.	The stream function is a mathematical concept.  It has no physical meaning.  Comment.

13.	Why is the Cd of an orificemeter considerably less than that of a venturimeter?

14.	Identify the two important functions of the volute casing of a centrifugal pump.

15.	Give the comparison between impulse and reaction turbines.

PART – C (5 x 15 = 75 MARKS)

16.	A 400mm diameter shaft is rotating @ 200 rpm. in a bearing of length 120mm.  If the thickness of oil film is 1.5mm and the dynamic viscosity of the oil is 0.7 Ns/m2.  Determine.

a.	Torque required to overcome friction in bearing.

b.	Power utilised in overcoming viscous resistance.  Assume a linear velocity profile.

(OR)

17.	An opening in a dam is covered by the use of a vertical sluice gate.  The opening is 2m wide and 1.2m high.  On the upstream of the gate the liquid of specific gravity 1.45, lies upto a height of 1.5m above the top of the gate, whereas on the downstream side the water is available upto a height touching the top of the gate.  Find 	a)	The Resultant force acting on the gate and position of centre of pressure	b)	The force acting horizontally at the top of the gate which is capable of opening it.	Assume that the gate is hinged at the bottom.

18.	a.	Mention two practical applications of the equation of continuity.					(2)

b.	Derive the continuity equation in Cartesian co-ordinates.							(13)

(OR)

19.	For a two-dimensional flow, the velocity function is given by the expression  = x2 – y2.

a.	Determine velocity components in x and y directions.								(5)

b.	Show that the velocity components satisfy the conditions of flow continuity and 	irrotationality.																(5)

c.	Determine stream function and the flow rate between the streamlines (2, 0) and (2, 2).(5)

[P.T.O]

20.	A horizontal venturimeter with inlet and throat diameter 300 mm and 100mm respectively is used to measure the flow of water.  The pressure intensity at inlet is 130kN/m2 while the vacuum pressure head at the throat is 350mm of mercury.  Assuming that 3% of head is lost in between the inlet and throat, Find   (a) The value of Cd (Co-efficient of discharge) for venturimeter and   (b) Rate of flow.

(OR)

21.	Three pipes of diameters 300mm, 200mm and 400mm and lengths 450m, 255m and 315m respectively are connected in series.  The difference in water surface levels in two tanks is 18m.  Determine the rate of flow of water if co-efficient of friction are 0.0075, 0.0078, 0.0072 respectively considering:

a.	Minor losses also and 		b.	Neglecting minor losses.

22.	a.	What is cavitation?  What are the effects of cavitation?							(5)

b.	A centrifugal pump working in a dock pump, 1565 l/s against a mean lift of 6.1m when the impeller rotates at 200 rpm.  The impeller diameter is 1.22 m and the area at outer periphery is 6450 cm2.  If the vanes are set back at an angle of 26 at the outlet, Determine

i)	Hydraulic efficiency 	ii)	Power required to drive the pump.					(10)

(OR)

23.	A jet of water strikes with a velocity of 35m/s a flat plate inclined at 30 with the axis of the set.  If the cross sectional area of the jet is 25 cm2.  Determine (a) the force exerted by the jet on the plate.   (b) The components of the force in the direction normal to the jet.	(c) The ratio in which the discharge gets divided after striking the plate.

24.	A pelton wheel is to be designed for the following specifications:

Power (brake of shaft)		- 9560 kW

Head					- 350m

Speed					- 750 rpm

Overall efficiency		- 85%

Jet diameter – not to exceed 1/6 th of the wheel diameter.  Determine the following:

a.	The wheel diameter		b.	Diameter of the jet	c.	The number of jets required

Take Cv = 0.985, speed ratio = 0.45.

(OR)

25.	A reaction turbine works at 450 rpm under a head of 120m.  Its diameter at inlet is 1.2m and the flow area is 0.4m2.   The angles made by absolute and relative velocities at inlet are 20 and 60 respectively with the tangential velocity.  Determine.

a.	 The volume flow rate		b.	The power developed		c.	The hydraulic efficiency