# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	ADVANCED DESIGN OF STEEL STRUCTURES		 Time: 3 hours

# Subject Code:	09CE304		         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	A non-sway column in a building frame with flexible joints is 4 m high and subjected to the 	following load and moment:

Factored axial load = 500 kN; Factored moment ‘MZ’ at top of column = 27 kNm and at 	bottom of column = 45 kNm.  Design a suitable beam-column assuming fy = 250 N/mm2.  Take 	the effective length of the column as 0.8L along both the axes.							(12)

b.	A proposed cantilever beam is built into a concrete wall.  It supports a dead load of 20 kN/m and a live load of 10 kN/m.  The length of the beam is 5 m.  Select suitable section with necessary checks.  Assume stiff bearing length of 100 mm.										(8)

(OR)

2.	Design a simply supported beam of 7 m span carrying reinforced concrete floor capable of providing lateral restraint to the top compression flange.  The total UDL is made-up of 100 kN dead load including self weight plus 150 kN imposed load.  In addition, the beam carries a point load at mid span made up of 50 kN dead load and 50 kN imposed load (assuming a stiff bearing length of 75 mm).

3.	Compulsory:

Design a purlin for an industrial building located at Guwahati with a span of 20 m and a length of 50 m. The roofing is galvanized iron sheeting.  Basic wind speed is 50 m/s and the terrain is an open industrial area.  Building is class B building with a clear height of 8 m at the eaves.  The span of purlin is 5 m and   spacing of purlin = 1.275 m.  Also perform the wind pressure analysis.

4.	a.	Define tower and bring out the difference between a tower and mast.						(2)

b.	Mention the types of tower as per IS 802 (Part I) 1967 and loads to be considered in the 	analysis 	of tower.				     														(6)

c.	Briefly explain the procedure adopted in the analysis of tower.    						(12)

(OR)

5.	Briefly explain the seismic behaviour of the following

a.	Bracing members  	b.	Beam column 		c.	‘I’ section  	d.	Rectangular hollow section.

6.	Analyse the building frame shown in figure below by using portal method.  The ends A, D, G and J are fixed ends.

15 kN

C

B

A

D

G

J

I

F

E

H

K

L

4 m

6 m

4 m

6 m

4 m

20 kN

(OR)

[P.T.O]

7.	Analyse the building frame shown in figure in problem 6 by cantilever method.

8.	A continuous beam ABCD is loaded as shown in figure below.  Determine the collapse load in case

a.	the beam is of uniform section and 	b.	the beam is of unequal section.

4 m

D

6 m

4 m

60 kN/m

80 kN/m

100 kN/m

A

B

C

(OR)

9.	Determine the value of plastic moment for the frame shown in figure below.  The plastic moment is uniform throughout.

100 kN

B

A

E

C

3 m

10 m

D

20 kN/m