# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	STRUCTURAL DYNAMICS							 Time: 3 hours

# Subject Code:	09CE308 		         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	Define the following

i)	Amplitude    ii)	Frequency  	iii)	Vibration	iv)	Degrees of freedom		(8)

b.	A vibration test was conducted on an empty elevated water tank. A cable attached to the tank applies a lateral horizontal force of 73kN and pulls the tank horizontally 50mm. The cable is suddenly cut and the resulting free vibration is recorded. At the end of four complete cycles, the time is 2.0 sec and the amplitude is 25mm. Compute the following (i)	Damping ratio  (ii)	Natural period of undamped vibration  (iii) 	Stiffness

(iv) Weight  (v) Damping Coefficient  (vi) Number of cycles required for the displacement amplitude to decrease to 5mm.									      (12)

(OR)

2.	a.	A single degree of freedom system has an undamped natural frequency of 1.10Hz and 	damping factor of 10%. The initial displacement is zero and the initial velocity is 0.5m/s. 	Determine the damped natural frequency and the equation of motion for the system.	(10)

b.	A steel rigid frame supports a rotating machine which exerts a horizontal force at the ground level 50,000sinωt where ω = 12. Assuming 5% damping, find the steady state amplitude of the vibration.														(10)

3.	Compulsory:

F(t)

K

X

t

M

F (t)

a.	An undamped simple oscillation shown in figure is acted by a periodic force shown 	in   	figure. Determine the response of the system.									(10)

t

2T

3T

T

b.	Determine the dynamic response of a tower subjected to blast loading. The idealization of the structure and the first loading are shown. Neglect damping.						(10)

F(t)

1200

0

0.02

0.04

0.06

t in sec

F (t)

W=30kN

K=100kN/m

4.	a.	Find the fundamental frequency of a uniform cantilever of length ‘l’ and a mass ‘m’ by 	Rayleigh method. Take the shape function as ψ(x) = 1- (cos πx/2l).					(10)

m

EI, mb

L/2

L/2

b.	Find the natural frequency of a simply supported beam of length ‘l’ and mass ‘m’ using Rayleigh’s method. Assume deflection curve as Ф (x) = y. sin (πx/l).				(10)

(OR)

[P.T.O]

5.	A simply supported beam of length ‘l’ and of constant cross section is executing simple harmonic motion. Find the values of natural frequencies of first 3 modes and sketch the mode shape.

6.	a.	State and prove the orthogonal property of normal modes.							(10)

b.	A simply supported beam of span 8m is subjected to a concentrated force of 700 N applied suddenly at a point 2m from the left end. Mass of the beam is 750kg/m and EI=30x106Nm2.  Determine the response by considering first two modes only.	 	(10)

(OR)

7.	Calculate the first mode shape and frequency of the three storeyed building shown in figure using Stodola method. The Flexibility matrix is as follows.

m =1.5

m = 2

m =1

8.	Consider a simple system for which the governing equations are a2 = 2.8,	T1=4.45. Calculate the response of the system.

(OR)

9.	a.	Explain briefly the step-by-step solution using Newmark Integration Method.		(10)

b.	Discuss how to idealize a multistoreyed building frame for dynamic analysis.		(10)