# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title: 	FLUID MECHANICS AND MACHINERY				       Time: 3 hours

# Subject Code:	CE212		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define Newton’s law of viscosity.

2.	State Hydrostatic law.

3.	Define steady and unsteady flow.

4.	Write the continuity equation.

5.	Define coefficient of discharge.

6.	Write the Darcy-Weisback equation.

7.	Define cavitation.

8.	Define the slip of reciprocating pump.

9.	What is specific speed of a turbine?

10.	What is the function of a surge tank?

PART – B (5 x 3 = 15 MARKS)

11.	What are the types and their units of viscosity?

12.	Differentiate laminar and turbulent flow.

13.	What are the minor losses in flow through pipes?

14.	What are the differences between reciprocating and centrifugal pump?

15.	What is the use of the draft tube?

PART – C (5 x 15 = 75 MARKS)

16.	An oil of viscosity 5 poise is used for lubrication between a shaft and sleeve. The diameter of the shaft is 0.5m and it rotates at 200 rpm. Calculate the power lost in oil for a sleeve length of 100 mm. The thickness of the oil film is 1 mm.

(OR)

17.	A pipe line which is 4 m in diameter contains a gate valve. The pressure at the centre of the pipe is 19.6 N/cm2, if the pipe is filled with an oil of specific gravity 0.87, find the force exerted by the oil upon the gate and position of centre of pressure.

18.	Derive the continuity equation in three dimensional coordinates.

(OR)

19.	The water is flowing through a pipe having diameters 20 cm and 10 cm at sections 1 and 2 respectively. The rate of flow through the pipe is 35 litres/sec. The section 1 is 6 m above datum and section 2 is 4 m above the datum. If the pressure at section at 1 is 39.24 N/cm2, find the intensity of pressure at section 2.

[P.T.O]

20.	The inlet and throat diameters of a horizontal venturimeter are 30 cm and 10 cm respectively. The liquid flowing through the meter is water. The pressure intensity at the inlet is 13.734N/cm2 while the vacuum pressure at the throat is 37 cm of mercury. Find the rate of flow. Assume that 4 % of differential is lost between the inlet and throat. Find also the value of coefficient of discharge for venturimeter.

(OR)

21.	The difference in water surface levels in two tanks, which are connected by three pipes in series of length 300 m, 170 m and 210 m and of diameters 300 mm, 200 mm and 400 mm respectively, is 12 m. Determine the rate of flow of water if coefficient of friction in the three pipes are 0.005, 0.0052 and 0.0048 respectively, considering (i) minor losses also, (ii) neglecting minor losses.

22.	Discuss principle and working of reciprocating pump with a neat sketch.

(OR)

23.	A centrifugal pump having outer diameter equal to 2 times of the inner diameter and running at 1000 r.p.m works against a total head of 40 m. The velocity of flow through the impellor is constant and equal to 2.5 m/s. The vanes are setback at an angle of 40º at outlet. If the outlet diameter of the impellor is 500 mm and width at the outlet is 50 mm. Determine (i) vane angle at the inlet (ii) work done by the impellor on water per second (iii) Manometric efficiency.

24.	A pelton wheel is to be designed for the following specification: Shaft power = 11.72 kW; Head = 380 meters; speed=750 r.p.m; Overall efficiency = 80%; Jet diameter is not to exceed one sixth of the wheel diameter, Determine (i)The wheel diameter (ii)The number of jets required (iii) Diameter of the jet. Take Kv1=0.985 and Ku1=0.45

(OR)

25.	A Francis turbine with an overall efficiency of 75% is required to produce 148.25 kW power. It is working under a head of 7.62 m. The peripheral velocity equal to 0.26(2gH)1/2 and the radial velocity of flow at inlet is 0.96(2gH)1/2. The wheel runs at 150 r.p.m. and the hydraulic losses in the turbine are 22 % of the available energy. Assume radial discharge, determine (i)guide blade angle (ii)the wheel vane angle at inlet (iii) diameter of the wheel at inlet (iv) Width of the wheel at inlet.