# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	MECHANICS OF DEFORMABLE BODIES - I			       Time: 3 hours

# Subject Code:	CE240		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define Young`s Modulus.

2.	What is the strain energy due to gradually applied axial load?

3.	Define Principal planes.

4.	What is the hoop stress in thin cylindrical shell?

5.	What is the maximum bending moment in a cantilever span ‘l’ carrying uniformly distributed load ‘w’ per unit run over the entire span?

6.	What are the various types of supports in a beam?

7.	What do you mean by flitched beam?

8.	Define section Modulus.

9.	What is the polar modulus of a circular section?

10.	What are the types of helical springs?

PART – B (5 x 3 = 15 MARKS)

11.	Draw the stress-strain diagram of mild steel.

12.	Write down the expression for principal stresses.

13.	What is the relationship between rate of loading, shear force and bending moment?

14.	Draw a neat sketch showing the variation of bending stress in the section of a beam.

15.	Write the Torsion equation.

PART – C (5 x 15 = 75 MARKS)

16.	Derive the relationship between Young`s Modulus ‘E’ and Rigidity Modulus `G’.

(OR)

17.	A concrete column of cross sectional area 400 mm x 400mm is reinforced by four longitudinal 50mm diameter round steel bars. If the column carries a compressive load of 300kN, determine:	a.	Loads carried by concrete and steel bars.	b.	 The stress produced in the concrete and steel bars.   	The Young’s modulus of steel is 15 times that of concrete.

18.	The principal stress at a point across two planes are 75 MN/m2 (tensile) and 35 MN/m2 (tensile). Determine the normal, tangential stress and the resultant stress and its obliquity on a plane at 20o with the major principal plane.

(OR)

19.	A cylindrical vessel whose ends are closed by means of rigid flange plates is made of steel 3mm thick. The internal length and diameter of vessel are 500mm and 250mm respectively. Determine the longitudinal and circumferential stress in the cylindrical shell due to an internal fluid pressure of 3 MN/m2. Also determine the increase in length, diameter and volume of the vessel. Assume Young`s modulus E = 200 GN/m2 and Poisson`s ratio 1/m =0.3.

[P.T.O]

20.	Draw the BM and SF diagrams for a cantilever loaded as shown in figure.

(OR)

21.	A beam AB 10m long carries an uniformly distributed load of 20kN/m over its entire length together with concentrated load of 50kN at left end and 80kN at right end. The beam is to be supported at two props at the same level, 6m apart so that the reaction is same at each support. Draw BMD and SFD and locate the point of contra flexure.

22.	A 250 mm x150mm rectangular beam is subjected to maximum bending moment of 750 kNm. Determine:

a.	The maximum stress in the beam.

b.	If the value of E for the beam material is 200GN/m2, find out the radius of curvature for 	that portion of the beam where the bending is maximum.

c.	The value of the longitudinal stress at a distance of 65mm from the top surface of the 	beam.

(OR)

23.	A laminated steel spring 1m long is to support central load of 5.8kN. If the maximum deflection of spring is not exceed 45mm and maximum stress should not exceed 300MN/m2, determine

a.	The thickness of the leaves.

b.	Their number if each plate is to be 80mm wide.	Take E=200GN/m2.

24.	A hollow shaft of diameter ratio 3/8 is required to transmit 600 kW at 110 r.p.m, the maximum torque being 20% greater than the mean. The shear stress is not to exceed 63 MN/m2 and the twist in a length of 3m not to exceed 1.4 . Determine the maximum external diameter satisfying these conditions. Assume Modulus of Rigidity C = 84 GN/m2.

(OR)

25.	A close coil helical spring has a stiffness of 10 N/mm. Its length when fully compressed with adjacent coils touching each other is 400mm. The modulus of rigidity of material of the spring C = 8 x 10 4 N/mm2.

a.	Determine the wire diameter and mean coil diameter, if their ratio= 1/10.

b.	If the gap between any two adjacent coils is 2mm, what maximum load can be applied 	before the spring becomes solid.

c.	What is the corresponding maximum shear stress in the spring?