# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	MECHANICS OF DEFORMABLE BODIES - II 			       Time: 3 hours

# Subject Code:	CE241		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define the term ‘deflection’ of a point in loaded beam.

2.	Write the expression for finding the maximum deflection of a cantilever beam of span ’l’  and subjected to uniformly distributed load of ‘w’ per m throughout the span.

3.	When is a column said to be a long column?

4.	State the mode of failure of short column subjected to axial compressive loading.

5.	What is the difference between thick cylinder and thin cylinder?

6.	In thick cylinder, name the stress that varies along the thickness of the cylinder.

7.	Define shear center.

8.	Define shear flow.

9.	When does unsymmetrical bending occur?

10.	The bending stress at a point (in unsymmetrical bending) will be positive or negative depending on _______.

PART – B (5 x 3 = 15 MARKS)

11.	Write the Mohr’s theorems connected with area moment method.

12.	A strut 2.5m long is 60mm in diameter. One end of the strut is fixed while the other end is hinged. Find the crippling load for strut using Euler’s formula. E = 2.21 x 105 N/mm2.

13.	Write the assumption made in the derivations of Lame’s equation.

14.	Write the Winkler Boch formula. Also write the expression for finding the position of neutral axis.

15.	Define product and principal moment of inertia.

PART – C (5 x 15 = 75 MARKS)

16.	A cantilever beam of 3m span is 150mm thick and 250mm deep. It carries a uniformly distributed load 20 kN/m over its full span and a point load of 25kN at the free end. Calculate the maximum slope and deflection. E = 210 x 109 N/m2.

(OR)

17.	A cantilever 2m long carries a load of 15kN at a distance of 1m from the fixed end and a load of 10kN at the free end. Determine the deflection at the free end. Use conjugate beam method. E = 200 x 106 kN/mm2. I = 15 x 10-6 m4.

18.	Compare the crippling loads given by Rankine’s and Euler’s formula for a tubular strut 2.25m long having outer and inner diameter of 37.5mm and 32.5mm loaded through pin-joint at both ends. Yield stress c = 315 x 106 N/m2    a = 1/7500; E = 200 x 109 N/m2.

(OR)

[P.T.O]

19.	Determine the crippling load for a T section of dimension 100 mm x 100 mm x 20 mm and of length 5m which is used as a strut with both of its ends hinged. Take young’s modulus             E = 2 x 105 N/mm2.

20.	A thick cylinder 125 mm inside diameter and 250 mm outside diameter is subjected to an internal fluid pressure of 50 N/mm2.  Calculate the maximum and minimum intensities of circumferential stress and sketch the distribution of circumferential stress intensity and radial pressure intensity across the section.

(OR)

21.	Determine the diameter of a bolt which is subjected to a pull of 9kN together with a transverse shear force of 4.5 kN using maximum principal stress theory.

22.	Determine the position of (a) neutral axis (b) maximum and minimum stresses when a curved beam of circular section of diameter 100mm is subjected to pure bending moment of          +11.5 kN - m. Radius of curvature of beam is 100m.

(OR)

23.	A curved bar is formed of a tube of 120 mm outside diameter and 7.5mm thickness. The center line of this beam is a circular arc of radius 225 mm. A bending moment of 3kNm tending to increase curvature of the bar is applied. Calculate the maximum tensile and compressive stresses set up in the bar.

24.	Determine the principal moment of inertia of the unequal angle section 90mmx60mmx10mm when 90 mm leg is placed vertical.

(OR)

25.	An 80mmx80mmx10mm angle section given below is used as a simply supported beam over a span of 2.4m. It carries a load of 400 N along the line YG where G is the centroid of the section. Calculate Stresses at the point A,B and C of the mid section of the beam                       E = 200 x 103 N/mm2

Y

A

Y

X

G

X

B

80mm

80mm