# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	MECHANICS OF FLUIDS 								       Time: 3 hours

# Subject Code:	CE245		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define Mass Density.

2.	State Pascal’s Law.

3.	Define Stream line.

4.	What is meant by rate of flow?

5.	What is meant by crest?

6.	Define Laminar Flow.

7.	What is Turbulence?

8.	Define hydraulic gradient line.

9.	What are dimensionless numbers?

10.	What is model analysis?

PART – B (5 x 3 = 15 MARKS)

11.	What is a Fluid? How are fluids classified?  Give examples of classification of fluid.

12.	Write briefly about continuity equation.

13.	Write short notes on Pitot tube and Venturimeter.

14.	What is Syphon? Where is it used?

15.	Explain the term dimensional Homogeneity.

PART – C (5 x 15 = 75 MARKS)

16.	A differential manometer is connected at the two points A and B in two pipes. The pipe A contains a liquid of specific gravity 1.5 while pipe B contains a liquid of specific gravity 0.90. The Pipe B lies 3m below the centre of the pipe A. The pipe A lies 5m above top of mercury level of limb A. The difference of mercury level between the two left limb is ‘h’ m in the left limb A. The pressures at A and B are 1 kgf/cm2 and 180 kgf/cm2 respectively. Find the mercury level difference ‘h’ in the manometer.

(OR)

17.	Derive the expression for total pressure and centre of pressure for a vertically immersed surface.

18.	State and derive Bernoulli’s equation. What are the assumptions made while deriving Bernoulli’s Equation?

(OR)

19.	Prove that in case of forced vortex, the rise of liquid level at the ends is equal to the fall of liquid level at the axis of rotation.

[P.T.O]

20.	A sharp crested rectangular weir of 1m height extends across a rectangular channel of 3m width. If the head of water over the weir is 0.45m, calculate the discharge. Consider velocity of approach and assume the value of Cd.

(OR)

21.	A rough pipe of diameter 400 mm and length 1000 m carries water at the rate of 0.4 m3/s. The wall roughness is 0.012 mm. Determine the co-efficient of friction, wall shear stress, centre-line velocity and velocity at a distance of 150 mm from the pipe wall.

22.	Derive an expression for head loss due to (a) Sudden enlargement and (b) Sudden contraction of a pipe.

(OR)

23.	Two reservoirs A and C having a difference of level of 15.5m are connected by a pipeline ABC the elevation of Point B being 4.0m below the level of water in reservoir A. The length AB of the pipeline is 250m, the pipe being made of mild steel having a friction coefficient f1 while the length BC is 450m, the pipe having made of cast iron having friction co efficient f2.  Both the lengths AB and BC have a diameter of 200mm. A partially closed valve is located in the length BC at a distance of 150m from reservoir C.

If the flow through the pipe line is 3m3/min, the pressure head at B is 0.5m and the head loss at the valve is 5m.

a.	Find the friction co-efficient f1 and f2	b.	Draw the hydraulic grade line of the pipe line

24.	Using Buckingham’s - theorem, show that the velocity through a circular orifice is given by

V =2gHΦ(D/H, µ/ρVH), where H is head causing flow, D is the diameter of the orifice, µ is the coefficient of viscosity,  is the mass density and g is the acceleration due to gravity.

(OR)

25.	Explain the dimensionless numbers in detail.