# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	MECHANICS OF SOILS 								       Time: 3 hours

# Subject Code:	CE247		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The dry density and density at 50 % saturation of a soil are 1.7g/cc and 1.9 g/cc respectively Find its porosity.

2.	What is the difference between sensitivity and thixotropy of clays?

3.	What is the difference between seepage velocity and discharge velocity?

4.	What are the factors affecting permeability.

5.	What is relative compaction? Where is it used?

6.	What is hydrodynamic lag?

7.	Find the capillary rise in a sandy soil which has a void of 0.65 and the effective size of particle is 0.07mm. Take C= 0.1cm2.

8.	What are the different types of settlement?

9.	What is deviator stress?

10.	What is the difference between finite slope and infinite slope?

PART – B (5 x 3 = 15 MARKS)

11.	List the various corrections that are required for a hydrometer reading. How are these corrections applied?

12.	What is piping?

13.	State the assumptions in Terzaghi`s one dimensional consolidation theory?

14.	What is the use of New marks Influence chart

15.	Find the factor of safety of an infinite slope of cohesion less soil of angle of   internal friction 36°, if the slope angle is 30°.

PART – C (5 x 15 = 75 MARKS)

16.	The in-situ void ratio of a granular deposit is 0.50.The maximum and      minimum voids ratio of the soil were determined to be 0.75 and 0.35 G=2.67.Determine the relative density and relative compaction of the    deposit.

(OR)

17.	One cubic metre of wet soil weighs 20kN.Its dry weight is 18 kN.  Specfic gravity of solids  is 2.67.  Determine the water content, bulk density, dry density, porosity, void ratio and degree of   saturation.

18.	A permeameter of 10cm dia and containing a soil sample of length 25 cm can be used   as a falling head and constant head apparatus. When a constant head is maintained at the top, a discharge of 5cc/sec is passed vertically downwards at head of 100cm.  Calculate the coefficient of permeability. While  using  the  same  permeameter  as  falling  head  one  and  testing the same soil, what  time interval should be expected for the fall from 1m to 60cm.  The diameter    of standpipe is 2cm.

(OR)

19.	Explain the properties, method of construction and applications of flow net.

[P.T.O]

20.	An earthen embankment of 106 m3 volume is to be constructed with a soil having void ratio of 0.8 after compaction. There are three borrow pits marked A,B and C, having soils with void ratios of 0.90, 1.50 and 1.80, respectively. The cost of excavation and transporting the soil is Rs 0.25, Rs 0.23 and Rs 0.18 per m3, respectively. Calculate the volume of soil to be excavated from each pit. Which borrow pit is the most economical. Assume G = 2.65.

(OR)

21.	A stratum of clay is 2m thick and has an initial overburden pressure of 50 kN/m2 at its middle. Determine the final settlement due to an increase in pressure of 40  kN/m2 at  the middle of the clay layer. The value of coefficient of compression index is 0.25.Take initial void ratio of 1.40.

22.	A layer of fine sand 3m in thickness rests on a bed of clay and the water table is at a depth of 2m below   the ground level. The porosity of sand is 50% and the degree of saturation of the sand above the water   table is 60% and the specific gravity of solids is 2.7.The sp. gravity of solids and water content of the clay are 2.72 and 40% respectively. Determine the effective stress at a depth of 7m from the ground level.

(OR)

23.	a.	A concentrated load of 800 kN acts at the ground surface. Compute the vertical stress 	at 8m depth and 2m away from the axis.

b.	An elevated structure with a total weight of 10,000 kN is supported on a tower with four legs. The legs rest on piers located at the corner of a square 6m side. What is the vertical stress due to this loading at a point 7m beneath the centre of the structure?

24.	Undrained triaxial tests are carried out on 4 identical specimens of clay and the following results are obtained

Cell Pressure (kN/m2)                        50                   100                  150                200

Deviator stress   kN/m2                    340                   430                  540                620

Pore pressure kN/m2                         4.8                      9                     11                  17

Determine the value of effective angle of shearing resistance and cohesion intercept by 	drawing failure envelope.

(OR)

25.	Prove from first principles that the factor of safety of an infinite slope of cohesion less soil is independent of the height of the slope.