# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	FOUNDATION ENGINEERING						      	       Time: 3 hours

# Subject Code:	CE248					              					       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What are the geophysical methods of soil exploration?

2.	How will you reduce the area ratio of the sampler?

3.	What are the different types of shear failure?

4.	Write down the Skempton’s formula for the determination of ultimate bearing capacity of soil.

5.	What are the different types of sheet pile wall?

6.	Define the coefficient of earth pressure at rest.

7.	What is deep foundation?

8.	Write down the Engineering New’s formulae.

9.	What is pneumatic caisson?

10.	What is raft foundation?

PART – B (5 x 3 = 15 MARKS)

11.	What are the requirements of foundation?

12.	What are assumptions made in Terzaghi’s bearing capacity theory?

13.	Define active earth pressure with diagrams.

14.	Define negative skin friction.

15.	What are the forces acting on the well foundation?

PART – C (5 x 15 = 75 MARKS)

16.	Define SPT number. Explain the standard penetration test and also write down the equations for corrections.

(OR)

17.	Explain the different types of foundations with neat sketch.

18.	A column carrying a load of 750 kN has to be supported by a square footing with its base at 1.5m depth. What is the required size of the foundation, which will provide a factor of safety against shear failure? Assume cohesion c=10 kN/ m2, ф = 30, γD=18 kN/ m3, γsub = 10 kN/ m3. Water table at 1.5 m depth, use Terzaghi’s equation.

(OR)

19.	A footing having a size of 1.8 x 3m has to transmit the load of a column at a depth of 1.5m.  Calculate the Safe load with the factor of safety =3. Soil has the following properties: porosity =10 %, specific gravity =2.51, cohesion, c = 8 kN/ m2, ф = 35. Use IS code equation.

20.	Explain Rehbann‘s Graphical Method of Earth Pressure theory with neat sketches.

(OR)

21.	A gravity retaining wall retains 12 m of a backfill, γ =17.7 kN/ m3, ф = 250 with a uniform horizontal surface. Assume the wall interface to be vertical, determine the magnitude and the point of application of total active pressure. If the water table is a height of 6 m, how far do the magnitude and the point of application of active pressure be changed? Draw the pressure diagram also.

[P.T.O]

22.	Explain the Classification of piles with neat sketches.

(OR)

23.	A ‘n’ pile has to be proposed in a uniform pattern of soft clay with equal spacing in all directions. Assuming the value of ‘c’,  determine the Optimum value of spacing of pile in the group. Take n =25, α = 0.7.  Assume the pile is circular. Neglect bearing resistance.

24.	Explain the elements of well foundations with neat sketch.

(OR)

25.	Proportion a Rectangular Combined Footing for the following data.

|  | load (kN) |
| --- | --- |
| column a | 600 |
| column b | 900 |

Allowable bearing pressure: 100 kN/ m2

Center to center spacing of columns = 5 m

Projection with heavier column is 1 m