# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	DESIGN OF STEEL STRUCTURES 						       Time: 3 hours

# Subject Code:	CE252		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

(Use of Relevant BIS codes and Steel Tables are permitted

Assume suitable data wherever necessary)

PART – A (10 x 1 = 10 MARKS)

1.	Define net area.

2.	What is lacing?

3.	Define web buckling.

4.	What is curtailment of plates?

5.	What is stiffener?

6.	What is splice?

7.	What is purlin?

8.	What is bent?

9.	What is light gauge steel section?

10.	What is a stiffened beam?

PART – B (5 x 3 = 15 MARKS)

11.	What is a column base?

12.	Explain lateral torsional buckling.

13.	Explain the concept of unstiffened seated connection.

14.	What are the forces to be considered in the design of gable rafter.

15.	Differentiate stiffened and unstiffened columns.

PART – C (5 x 15 = 75 MARKS)

16.	Design a tension member to support a load of 300 kN.  The length of the member is 3.6m.

(OR)

17.	Design a continuous strut to carry a load of 175 kN.  The effective length is 5.8m.

18.	A beam of span 9m carries 12 kN/m load.  Design the beam with built up section and restrict the depth to 400 mm.

(OR)

19.	Explain the design procedure of a beam subjected to biaxial bending.

20.	Design a welded plate girder of 30m span.  It is subjected to an UDL of 32 kN/m.  Design also the stiffeners and their connections.

(OR)

21.	An ISMB550 transmits an end reaction of 150kN and an end moment of 125 kNm to the flange of a column ISHB 250.  Design a welded connection.

[P.T.O]

22.	Design a gantry girder for the following data:

Crane load lifting capacity – 				300 kN

Weight of crane excluding trolley – 		180 kN.

Weight of trolley –						 75 kN

Minimum approach of crane hook – 		1.2m.

Distance between centres of gantry girders – 24m

Distance between centres of crane wheels – 	3m

Span of gantry girder –					 8m

Weight of rail section – 					0.4 kN/m

Height of rail – 							75mm.

(OR)

23.	Explain the design procedure of design of roof trusses.

24.	Explain the design procedure of unstiffened beams.

(OR)

25.	A self supporting chimney is 90m high and its diameter at top is 3m.  Design the plate for the stack.  Adopt wind pressure as per IS 875.  The location of the plate is such that the intensity of the wind pressure upto 40m height is 1.5kN/m2.