# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	SANITARY ENGINEERING 							       Time: 3 hours

# Subject Code:	CE253		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define Sewerage.

2.	What are colloidal solids in sewage?

3.	Name the various methods for disposal of screenings.

4.	Mention the factors on which the rate of settling of suspended solids depends.

5.	The trickling filter is always preceded by ________________ so that the settleable solids in the sewage may not clog the filter.

6.	Name the two types of air diffusers commonly used.

7.	Mention the different stages of sludge digestion. Indicate what happens to the pH of sludge during these stages.

8.	What is sewage sickness?

9.	What do you mean by back-siphonage?

10.	Which system of plumbing is most commonly used in India?

PART – B (5 x 3 = 15 MARKS)

11.	Enumerate the factors on which the cross-section of sewers would depend.

12.	The presence of floating matters such as oil and grease interfere with various treatment operations and hence should be removed from the sewage.  Indicate how these cause difficulties in treatment operations.

13.	Indicate with the help of a neat sketch the three different phases of the general growth pattern of bacteria in a culture.

14.	In waste water treatment, different types of sludges are generated at the treatment plants.  List the treatment units that generate sludge and the types of sludge.

15.	What are traps? On what factor does the efficiency of traps depend?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Discuss the relative advantage and disadvantages of the various sewerage systems.		(6)

b.	Design an outfall sewer in the separate system for a town with a population of 125,000 persons with water supply at 180 lpcd.  The sewer is to be of brickwork rendered smooth with cement mortar (n = 0.012) and the permissible slope is 1 in 1,000.  A self-cleansing velocity of 0.75 m/s is to be developed.  The dry-weather flow may be taken as one-third of the maximum discharge.  For discharge ratio of 1/3, take the depth ratio as 0.4 and velocity ratio as 0.88.																		(9)

(OR)

17.	a.	Enumerate and discuss the various factors affecting quantity of sanitary sewage.			(6)

b.	Write short notes on the following:

i)	BOD		ii)	COD		iii)	Stability of sewage						   (3x3)

[P.T.O]

18.	a.	With relevant sketches, describe the various types of screens.							(9)

b.	What are skimming tanks?  What are the salient design features?  What are the purposes of aeration in skimming tanks?														(6)

(OR)

19.	a.	Design a circular sewage sedimentation tank for a town having a population of 40,000.  The 	average water demand is 140 litres/capita/day.  Assume that 70% water reaches at the 	treatment unit and the maximum demand is 2.7 times the average demand. (Note: Provide 	an effective of 3 m in the first trial).													(9)

b.	With the help of sketches, describe the essential features of a grit chamber.  Discuss the methods of improving the efficiency of a grit chamber even during heavy fluctuations in quantity of sewage.																(6)

20.	a.	Sketch and describe the construction and working of a septic tank.						(8)

b.	Enumerate and discuss the various organic loading parameters to be considered for designing an activated sludge process.												(7)

(OR)

21.	a.	Sketch and describe the working of a standard trickling filter for purification of sewage.   (10)

b.	Enumerate the methods of aerating the mixed liquor and describe in brief any one system.	(5)

22.	a.	Explain briefly the process of sewage disposal by dilution.  Enumerate the conditions 	favouring dilution.																(7)

b.	With the aid of a neat sketch explain the oxygen sag curve of a stream into which sewage is discharged.																		(8)

(OR)

23.	a.	Design a sludge digestion tank with the following data:								(9)

Average flow of sewage = 16 x 106 litres per day

Total suspended solids in the raw sewage = 350 mg/l

Volatile suspended solids = 250 mg/l

Moisture content of the digested sludge = 87%

Sludge removed in the primary settling tank = 65%

Moisture content in the fresh sludge = 95%

Digestion period = 30 days

Provide Depth of sludge digestion tank = 6 m

b.	Write a short note on sludge disposal.												(6)

24.	Enumerate the various types of sanitary fittings and describe any four of them with sketches.

(OR)

25.	a.	With the help of neat sketches, describe the various types of traps.						(10)

b.	With the help of sketch, illustrate the single-stack partially ventilated plumbing system.	(5)