# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	MECHANICS OF FLUIDS 								       Time: 3 hours

# Subject Code:	CE277		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define Pascal’s Law.

2.	Mention any two units in which intensity of pressure is measured.

3.	Define forced vortex flow.

4.	What is meant by potential function?

5.	Name the devices used for measuring flow through pipes.

6.	What is meant by broad crested weir?

7.	Distinguish between Laminar flow and Turbulent flow.

8.	Define hydraulic gradient.

9.	Give dimensions in MLT System for Dynamic viscosity and pressure.

10.	List the various dimensionless numbers.

PART – B (5 x 3 = 15 MARKS)

11.	What is a manometer? What are the various types of manometer?

12.	What do you mean by stream line and streak line?

13.	Explain the principle of Pitot tube.

14.	Distinguish between energy gradient line and hydraulic gradient line.

15.	What is meant by distorted model and undistorted model.

PART – C (5 x 15 = 75 MARKS)

16.	A square plate 1.8m side and 1.8 mm thick weighing 60 N is to be lifted through a vertical gap of 30 mm of infinite extent. The oil in the gap has a specific gravity of 0.95 and the viscosity of 3N-s/m2. If the metal pate has to be lifted at a constant speed of 0.12 m/s, find the force and power required.

(OR)

17.	a.	Derive an expression for Total pressure and centre of pressure for a vertical plane surface 	submerged in a liquid															(10)

b.	Determine the total pressure on a circular plate of diameter 1.5 m which is paced vertically in water in such a way that the centre of the plate is 3m below the free surface of water. Find the position of centre of pressure.											(5)

18.	a.	Derive an expression for three dimensional continuity equation.					(10)

b.	In two dimensional potential flow, the velocity potential function is given by

Φ = 4x (3y-4).  Determine the value of stream function at point (3, 4)                 		(5)

(OR)

19.	a.	The water is flowing through a tapering pipe having diameters 300 mm and 150 mm at    	section 1 and section 2 respectively. The discharge through the pipe is 40 litres/s. The 	section 1 is 10 m and section 2 is 4m above the datum. Find the intensity of pressure at 	section 2, if that at section 1 is 400 kN/m2.										(10)

b.	State Bernoulli’s equation and List the assumptions for the derivation of Bernoulli’s equation.										 							(5)

[P.T.O]

20.	a.	Derive an expression for determination of discharge through a venturimeter. 		(10)

b.	A vertical sharp edged orifice of 100 mm diameter is discharging water at a rate of

95 liters/s under a constant head of 8 m. A point in the jet is measured from the venacontracta of the jet has co-ordinates 4.5 m horizontal and 0.54 m vertical. Find the coefficient of velocity, coefficient of discharge and coefficient of contraction.		(5)

(OR)

21.	a.	The following data relate to an inclined venturimeter, diameter of pipe = 400 mm, diameter 	of throat = 200 mm, inclination of pipe with the horizontal =30º, the distance between 	mouth and throat of the meter = 600mm, specific gravity of oil flowing = 0.7, specific 	gravity of mercury =13.6, differential manometer reading =50 mm, Cd= 0.98, determine 	the 	rate of flow.							      									(8)

b.	Find the discharge through a rectangular orifice 2 m wide and 1.5 m deep fitted to a water tank. The water level in the tank is 3.0 m above the top edge of the orifice, Take Cd =0.62.																			(7)

22.	a.	Derive an expression for Hagen Poiseulle equation for a circular pipe.	      			(10)

b.	A horizontal pipe carries water at a rate of 40 litres/sec, Its diameter, which is 0.3m reduced abruptly to 0.15 m. Calculate the pressure change across the contraction if the coefficient of contraction is 0.62 and head loss due to contraction is 0.0978m.      					(5)

(OR)

23.	A lubricating oil of viscosity 1 poise and specific gravity 0.9 is pumped through a 30mm diameter pipe.  If the pressure drop /m length of the pipe is 20kN/m2, find the i) flow rate, ii)shear stress@ the pipe wall, iii)  Reynolds number, iv) Power required /50m length of the pipe to maintain the flow.

24.	The resisting force of a supersonic aircraft during its flight depends on the variables such as length, velocity, viscosity, density and bulk modulus.  With the help of Rayleigh’s theorem, derive an expression showing the relationship between resisting force and these variables.

(OR)

25.	Obtain scale ratios for velocity, time, acceleration and force for the models governed by equality of Froude’s number.