# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	DIGITAL INTEGRATED CIRCUITS 					       Time: 3 hours

# Subject Code:	09EC214	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Subtract 12 from 7 using 2’s complement method.

2.	(1100111.110)2 = (        )8

3.	What is the equivalent gate for the logical diagram shown in the figure?

4.	What are the methods used for Boolean minimization?

5.	What is the 7400 series number coding for NOR gate?

6.	Draw the equivalent circuit diagram for a NOT gate.

7.	How to make Johnson counter from a ring counter?

8.	What are flip-flops?

9.	Define DRAM.

10.	What is meant by secondary memory?

PART – B (5 x 3 = 15 MARKS)

11.	Define excess 3 codes.

12.	Draw the half adder circuit.

13.	Define TTL.

14.	Draw the JK flip flop logic diagram.

15.	Define EPROM.

PART – C (5 x 15 = 75 MARKS)

16.	Design a full adder and draw its logical circuit diagram.

(OR)

17.	a.	Convert (0.513)10 into octal		b.	Convert (673.124)8 into binary

c.	Using 9’s complement subtract 3250 from 72532.

18.	Reduce the expression  F = Σm(0, 1, 2, 3, 6, 7, 13, 15) by mapping and implement in NAND 	logic.

(OR)

19.	Design and construct a 8:1 Multiplexer.

20.	Explain with neat circuit diagram the three state TTL devices.

(OR)

21.	Discuss the CMOS characteristics.

22.	Explain the operation of Synchronous up counter with neat diagram.

(OR)

23.	Describe all the modes of 4 bit shift register with neat diagram.

24.	Explain the 64 bit RAM with neat logical diagram.

(OR)

25.	Explain in detail different types memory.