# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	COMMUNICATION THEORY AND SYSTEMS 			       Time: 3 hours

# Subject Code:	09EC222	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define Modulation.

2.	Define the term ‘Bandwidth’.

3.	What is critical modulation?

4.	Mention the need for suppression of carrier.

5.	What is coherent detection?

6.	Write the mathematical expression for AM signal.

7.	What is image frequency signal?

8.	Write the limitations of TRF receiver.

9.	Define the term ‘noise’.

10.	Define noise figure.

PART – B (5 x 3 = 15 MARKS)

11.	Compare base band and pass band signals.

12.	A transmitter radiates 9 kW without modulation and 10.125 kW after modulation. Determine the depth of modulation.

13.	Draw the block diagram for generating single sideband suppressed carrier (SSBSC) using phase shift method.

14.	Write short notes on image frequency signals and their rejection.

15.	Calculate the overall noise figure of a three stage cascaded amplifier, each stage having a power gain of 10dbs and a noise figure of 6dbs.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Draw the block diagram of a basic communication system and explain each block. 		(5)

b.	Why is modulation essential for communication? 									(5)

c.	Write any five advantages of wireless communication. 								(5)

(OR)

17.	Explain the different types of modulation in detail.

18.	a.	Derive the mathematical expression for Amplitude modulated signal and obtain its 	frequency spectrum. 																(8)

b.	Derive the expression for total power carried in AM transmitter. 						(7)

(OR)

19.	a.	With a neat diagram derive an expression for obtaining PM from FM. 					(7)

b.	Compare AM, FM, and PM .														(8)

20.	a.	With a neat circuit diagram, explain square law diode modulator.						(8)

b.	Explain the generation of FM by direct method. 										(7)

(OR)

[P.T.O]

21.	Explain Foster Seeley discriminator with neat circuit and phasor diagrams.

22.	a.	With neat block diagram, explain low level and high level transmitter. 					(8)

b.	Draw the block diagram of SSB transmitter and explain it in detail. 					(7)

(OR)

23.	Draw the block diagram of super heterodyne receiver and explain each block.

24.	Discuss on various types of noises in detail.

(OR)

25.	Derive the expression for calculating the output signal to noise ratio for SSB/SC Signal.