# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination –  April/May 2011

# Subject Title:	ADVANCED RADIATION SYSTEMS 					 Time: 3 hours

# Subject Code:	09EC301		         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Explain the radiation through an aperture in an absorbing screen.				(10)

b.	State and explain Babinet’s principle with an example. 						(10)

2.	a.	Develop expression for the fields radiated by a current element.				(15)

b.	The radial component of the radiated power density of an antenna is given by

Wrad=ar A0 sinθ/r2 (W/m2), where A0 is the peak value of power density. Determine the total radiated power.														(5)

(OR)

3.	a.	Derive the radiated fields from a half wave dipole. 							(10)

b.	Derive expression for the power radiated in the far field and radiation resistance of the half wave dipole.															(10)

4.	Define uniform linear array. Derive expressions for the field of an N element broadside array. Obtain expressions for maxima and null directions and the beam width.

(OR)

5.	a.	For a four element end fire array with the elements placed along the Z axis at a distance, 	d= λ/2 apart with the maximum of array factor directed towards θ=0°, find the 	progressive phase excitation between the elements and the angles (in degrees) where the 	nulls of the array factor occur.												(10)

b.	What are Binomial arrays? Design a Binomial array with four isotropic sources placed symmetrically along the z-axis at a distance of d apart. 						(10)

6.	a.	What are the types of Horn antenna? Explain the basic principle and obtain the 	expressions for fields radiated by a pyramidal horn antenna.

(OR)

7.	a.	Design a rectangular micro strip antenna using a substrate with dielectric constant of 2.2, 	h=0.1588cm so as to resonate at 10GHz.						 			(10)

b.	Explain the working principle of Reflector antennas with neat diagrams.		(10)

8.	a.	Explain the types of polarization.											(10)

b.	Describe the Poincare representation of wave polarization. 					(10)

(OR)

9.	a.	The electric field of a linearly polarized electromagnetic wave given by

Ei = ax E0(x,y) e-jkz is incident upon a linearly polarized antenna whose electric field 	polarization can be 	expressed as Ea = (ax+ay) E(r,θ, φ). Find the polarization loss 	factor in dB. Comment on 	the result.										(10)

b.	What are Stokes parameters? Give its properties.								(10)