# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	VLSI DIGITAL SIGNAL PROCESSING 				 Time: 3 hours

# Subject Code:	10EC312		         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Design and implement a parallel processing architecture for a 3 tap FIR filter with block 	size 3.																		(10)

b.	What is the impact of combining pipelining and parallel processing on the sampling period? Comment on the same with relevant diagrams. What is fine grain pipelining?		(10)

2.	a.	With the DFG given discuss the application of unfolding in sample period reduction 	(10)

b.	Unfold the following DFG by factor 3											(10)

(OR)

3.	a.	Design Parallel FIR filters by transposition. Discuss the parallel filtering algorithms from 	linear convolutions. 					             	      								(15)

b.	State and prove the properties of Retiming.	                           	       					(5)

4.	Mention the need for fast convolution algorithms. Write the algorithm for Cooktoom, Modified Cooktoom, Winograd Algorithm and Iterative Convolution Algorithm.

(OR)

5.	Explain the method of Systolic array design and design B2. Also draw the low level implementation, block diagram and space time representation of design B2.

6.	Write short notes on:

a.	CSD Representation and CSD Multiplication.    									(10)

b.	Design a Lyon’s bit serial multiplier using Horner’s Rule.							(10)

(OR)

7.	a.	Using state variable description of digital filter, derive Lyapunov equation.  			(10)

b.	Design a 4 x 4 array multiplier and discuss its features.		      	  				(10)

[P.T.O]

8.	a.	Perform multiple constant multiplication for set of constants a = 237, b= 182, c= 93 using 	iterative matching algorithm.						         						(10)

b.	Perform subexpression sharing for the given constant coefficient (in CSD Representation) 4 tap FIR filter given below												     	        (10)

Y(n) = 1. 01010000010*x(n) +  0.10001010101*x(n-1) +0.10010000010 *   x(n-2) + 	1.00000101000 * x(n-4)

(OR)

9.	Write short notes on synchronous and wave pipelining. 							(10+10)