# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	LOW POWER VLSI DESIGN 							 Time: 3 hours

# Subject Code:	10EC314		         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Bring out the need for low power VLSI chip.									(6)

b.	A 32 bit off chip bus operating at 5V and 66MHz clock rate is driving a capacitance of 	25pF/bit. Each bit is estimated to have a toggling probability of 0.25 at each cycle. What 	is the power dissipation in operating the bus?									(4)

c.	Explain in detail the steps involved in simulation based gate level timing analysis.	(10)

2.	a.	Explain the transistor network restructuring and reorganization of CMOS complex logic 	gate to compute the Boolean function Y=A(B+C).								(15)

b.	Write short notes on CMOS floating node.										(5)

(OR)

3.	a.	Draw the hardware architecture of a synchronous state machine. Also discuss the 	transition analysis of state encoding.											(10)

b.	Describe the basics of pre-computation logic. Explain the binary comparator function using pre-computation logic.													(10)

4.	a.	Given y=x1x2+x1x3, where xi, i=1,2,3 are mutually independent. If z=x1x2+y. Determine 	P(y) and P(z) without suppressing the exponent and after suppressing the exponent.	(8)

b.	Derive the activities of static CMOS from signal probabilities.						(12)

(OR)

5.	a.	With the help of flow chart, explain how Monte-Carlo based technique is used for 	estimating power in both sequential and combinational circuits.					(10)

b.	Distinguish between static hazard and dynamic hazard with example.				(6)

c.	Write down the significance of estimation of Glitching power.						(4)

6.	a.	Explain about the leakage mechanism and different types of leakage current in deep sub-	micrometer transistor. Also discuss leakage current estimation.						(16)

b.	What is aspect ratio? Derive the aspect ratio for a MOSFET.						(4)

(OR)

7.	a.	Explain the steps involved in the multiple supply voltage design with any one example.																					(10)

b.	Describe SRAM cell operation with the help of 6T SRAM cell.					(10)

8.	a.	Explain the operation of adiabatic adder with the help of necessary circuit diagram. Also 	explain how these adiabatic circuits have constant energy dissipation.				(12)

b.	Draw the circuit diagram of Row driver circuit and explain its operation.			(8)

(OR)

9.	a.	Briefly explain the need for low power latches and flip flops.						(10)

b.	Discuss in detail regarding the evolution and quality measures for Latches.			(10)