# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	SIGNALS AND SYSTEMS 								       Time: 3 hours

# Subject Code:	EC227		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define periodic signals.

2.	List any two properties of convolution.

3.	When is a periodic signal said to have half-wave symmetry?

4.	Define fourier transform of a signal x(t).

5.	What is an anti-aliasing filter?

6.	Define Nyquist rate.

7.	What is the sufficient condition for the existence of DTFT?

8.	Define frequency response of a discrete time system.

9.	What is the Z-transform of the unit step sequence?

10.	Name any two methods of evaluating inverse Z-transform.

PART – B (5 x 3 = 15 MARKS)

11.	Check whether the system y(n) = cos[x(n)] are

a.	static or dynamic		b.	linear or non linear and 	c.	time invariant or time variant.

12.	State Parseval’s theorem for continuous time fourier series.

13.	Determine the transfer function of a zero order hold.

14.	Explain any three properties of DTFT.

15.	Find the Z-transform of the signal x(n)=[sinωon]u(n).

PART – C (5 x 15 = 75 MARKS)

16.	Find the response of the system with difference equation y(n)+ 2y(n-1) + y(n-2) = x(n) + x(n-1) for the input x(n) = (1/2)n u(n) with initial conditions y(-1) = y(-2) = 1.

(OR)

17.	a.	Find the output of a system with impulse response h(t) = e-3tu(t) and the input signal 	x(t)=u(t-3) – u(t-5).						        								(10)

b.	Give the procedure for Matrix convolution.					          				(5)

18.	Find the fourier series coefficients for the continuous time periodic signals.

x(t) =  1.5   for 0 ≤ t < 1

= -1.5   for 1 ≤ t < 2 	      with fundamental frequency Ωo = π.

(OR)

19.	State and prove the different properties of CTFT.

20.	Explain the reconstruction of band limited signals from its samples. Derive the necessary equations and draw the waveforms.

(OR)

21.	Describe the different sampling methods of continuous time signals.

[P.T.O]

22.	a.	Find the Fourier transform of the signal x(n) = (a)|n|.			           				(7)

b.	Find the convolution of the signals using Fourier transform:

x1(n) = (1/2)n u(n);	x2(n) = (1/3)n u(n)										(8)

(OR)

23.	Find the frequency response of a first order system described by difference equation

y(n) = ay(n-1) + x(n). Plot magnitude and phase response for a = 0.5.

24.	Solve the difference equation using Z-transform:

y(n) =  0.5y(n-1) + x(n),  n≥0

where x(n) = cos(π/3)n u(n);   y(-1) = 0.

(OR)

25.	State and prove any five properties of Z-transform.