# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	MICROWAVE AND OPTICAL COMMUNICATION ENGINEERING

# Time: 3 hours

# Subject Code:	EC244		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is the use of waveguide tee?

2.	Write the S matrix of an isolator.

3.	Define mode of a Reflex Klystron.

4.	What is the function of the helix in a TWTA?

5.	What are the modes of operation of a Gunn diode?

6.	Define VSWR in terms of reflection coefficient.

7.	Define Numerical Aperture of a optical fibre.

8.	Which wavelengths are called the optical windows?

9.	What is population inversion?

10.	What is quantum efficiency in a LED?

PART – B (5 x 3 = 15 MARKS)

11.	A waveguide termination having VSWR of 1.1 is used to dissipate 100 watts of power.  Find the reflected power.

12.	Compare TWTA and Klystron amplifier.

13.	Mention the important features of the Gunn diode.

14.	What is dispersion?

15.	List the important characteristics of a photo detector.

PART – C (5 x 15 = 75 MARKS)

16.	Explain the functioning of a directional coupler and derive its S matrix.

(OR)

17.	a.	Explain the operation a 3 port circulator.										(8)

b.	A three port circulator has an insertion loss of 1dB, isolation of 30 dB and VSWR = 1.5.  Find the S matrix.															(7)

18.	a.	With a neat diagram, describe the operation of a two cavity Klystron amplifier.		(10)

b.	Derive the efficiency of the two cavity Klystron and obtain its maximum efficiency.	(5)

(OR)

19.	Describe the mechanism of operation of a magnetron.  Derive equations for electron trajectory and Hull cut off voltage.

20.	What is a avalanche transit time effect?  Describe the construction and principle of operation of an IMPATT diode.

(OR)

21.	a.	Explain impedance measurement by slotted line method.							(8)

b.	Explain the wavemeter method to measure frequency.								(7)

[P.T.O]

22.	a.	Describe the theory of light propagation in a optical fibre.							(7)

b.	Bring out the differences between step and graded index and mono and multimode fibres.

(8)

(OR)

23.	Explain the various factors which influence the attenuation of light in optical fibre.

24.	a.	Describe the construction and principle of operation of a semiconductor LASER.	(10)

b.	Compare characteristics of LED and LASER.									(5)

(OR)

25.	a.	Describe the principle involved in optical detection in an avalanche photo detector.	(10)

b.	List the characteristics of PIN and APD photo detectors.							(5)