# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title : 	DIGITAL SIGNAL PROCESSING						      Time : 3 hours

# Subject Code:	EC245		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define shift-invariant system.

2.	What are the advantages of FFT over direct computation of DFT?

3.	What are the desirable features of FIR filters?

4.	Write the expression for Hanning window sequence.

5.	What are the advantages of Bilinear transformation?

6.	Give the expression for the location of poles of normalized Butterworth filter.

7.	Why is rounding preferred to truncation in realizing digital filters?

8.	What are the quantization errors due to finite word length registers in digital filters?

9.	Give an example each for fixed point and floating point digital signal processor.

10.	What are the different stages of pipelining in a processor?

PART – B  (5 x 3 = 15 MARKS)

11.	Obtain linear convolution of the sequences   using circular convolution.

12.	Show that the filter is a linear phase filter.

13.	Why do IIR filters not have linear phase?

14.	Express “7/8” and “-7/8” in sign-magnitude, 2’s complement and 1’s complement.

15.	What are the advantages and disadvantages of VLIW architecture?

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	A discrete time system is characterized by the equation

Check whether the system is an LTI system.				      						(6)

b. 	Given  Find  using DIT- FFT algorithm.			(9)

(OR)

17.	a. 	State and prove Parseval’s theorem for discrete time Fourier transform.        			(5)

b.   	Find the frequency response  and impulse response  of a causal discrete time LTI system which is characterized by the difference equation                                             				(10)

18.	A bandpass FIR filter of length 7 is required. It is to have lower and upper cut-off frequencies of 3 kHz and 6 kHz respectively and is intended to be used with a sampling frequency of 24 kHz. Determine the filter coefficients using Hanning window. Consider the filter to be causal.

(OR)

19.	 Explain how the design of linear phase FIR filters is done by frequency sampling method.

[P.T.O]

20.	The specifications of the desired low pass filter is

Design a Butterworth digital filter using impulse invariant transformation.

(OR)

21.	a. 	Apply the bilinear transformation for  with  and find 	     			     		                 										(6)

b. 	The normalized transfer function of an analog filter is given by. Convert the analog filter to a digital with a cutoff frequency of using bilinear transformation. 			      													(9)

22. 	Find the output round off noise power for the following transfer function

where  and  and  .

(OR)

23.	a.	Explain the characteristics of a limit cycle oscillation with respect to the system     	described by the difference equation y(n) = 0.95 y(n-1) + x(n). Determine the                	dead band of the filter. Assume 4-bit sign-magnitude representation excluding sign bit. 	Also assume 	                                                     				(10)

b. 	Draw the product quantization noise model of second order IIR system. 	      		(5)

24.	a. 	Distinguish between general purpose and special purpose digital signal processors.	(4)

b. 	Explain the factors that influence the selection of a DSP processor for a given application.									       												(8)

c. 	List the various on-chip peripherals in a digital signal processor.                    		(3)

(OR)

25.	Describe the concept of LMS adaptive algorithm with necessary mathematical expressions.