# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	VLSI DESIGN 								      	   		       Time: 3 hours

# Subject Code:	EC255		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define full custom design.

2.	What are the two common approaches used for Oxidation Process?

3.	Define threshold voltage of a MOS transistor.

4.	Write the expression for gm (linear) for MOS transistor.

5.	Draw the stick diagram for CMOS Inverter.

6.	Define sheet resistance.

7.	What is the advantage of Dynamic logic?

8.	Draw the simple two-phase clock generator circuit.

9.	What do you mean by systolic array?

10.	What are two techniques used for DRAM fabrication?

PART – B (5 x 3 = 15 MARKS)

11.	Discuss about physical design in VLSI design process.

12.	Write short notes on: a) Fowler Nordheim Tunneling b) Mobility Variation.

13.	Draw the layout for 2-input CMOS NAND gate.

14.	Explain about clocked CMOS Logic.

15.	Compare SRAM and DRAM.

PART – C (5 x 15 = 75 MARKS)

16.	Explain the different steps of fabrication of SOI (Silicon on Insulator) giving a schematic for every step.

(OR)

17.	Explain the following semi custom layout styles.

a.	Gate array layout 		b.	Macro cell layout		c.	Standard cell layout.			(3x5)

18.	Describe in detail the various regions in CMOS inverter DC characteristics.

(OR)

19.	With neat characteristics curve derive the expression for drain current in non saturated and saturated region.

20.	Obtain the expression for area capacitance and also explain about wiring capacitances.

(OR)

21.	Explain the λ-based design rules for n – well CMOS process with neat schematic.

22.	a.	Design a 4 to 1 multiplexer using nMOS switches. 	    						 		(8)

b.	Design a four-bit dynamic shift register using nMOS pass transistor and CMOS transmission 	gate   elements.													     		        	(7)

(OR)

[P.T.O]

23.	Explain about domino CMOS logic and design the CMOS domino logic for the function

Z= A.B + (C+D) (E+F) + G.H

24.	Explain in detail the design of an ALU Subsystem using a basic Adder.

(OR)

25.	Design a high speed carry look ahead adder using CMOS structures.