# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	PULSE AND WAVE SHAPING CIRCUITS 				       Time: 3 hours

# Subject Code:	EC289		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The voltage across the capacitor cannot change instantaneously. (True/False)

2.	Give the condition under which the low pass RC circuit acts as an integrator.

3.	Which condition is to be satisfied to avoid loading problem?

4.	Define transition time.

5.	Define Pulse width.

6.	Mention any two applications of Astable Multivibrator.

7.	Give the relationship between es,ed and et.

8.	What is the frequency of a television horizontal sweep circuit?

9.	Mention the types of Astable Blocking Oscillator.

10.	The pulse width of base timing Monostable Blocking Oscillator gets affected due to __________ replacement.

PART – B (5 x 3 = 15 MARKS)

11.	State Clamping theorem.

12.	Draw the transfer characteristics of Schmitt trigger and define hysteresis width.

13.	Write the base and collector voltage equation for a stable state collector coupled Monostable Multivibrator. ( Q1 off, Q2 on).

14.	Define and derive the expression of sweep speed for exponential charging.

15.	Give the advantages of unidirectional diode gate.

PART – C (5 x 15 = 75 MARKS)

16.	With neat sketch, explain the high pass RC network response to

a.	Pulse input		b.	Ramp input

(OR)

17.	With necessary circuits, explain the operation of Negative and Positive clamper.

18.	Neglecting the initial voltages, calculate the stable state current and voltages for the fixed bias flip flop circuit using npn transistors. Assume that the transistors have minimum hfe value of 20 and the state to be Q1 off, Q2 on.

[ Rc1 = Rc2 = 2.2 k, R1 = 14 k, R2 = 100 k, VCC = 12 V, VBB = - 12 V ]

(OR)

19.	With circuit diagram and waveform, explain the operation of Schmitt trigger using two transistors for a sinusoidal input.

20.	Explain the working principle of collector coupled Astable Multivibrator and derive its pulse width.

(OR)

21.	a.	Discuss the advantages and disadvantages of emitter coupled Astable Multivibrator.	(7)

b.	Define and derive the magnitude of the overshoot for collector coupled Monostable 	Multivibrator.																(8)

[P.T.O]

22.	Explain the operation of Exponential sweep circuit and derive the expressions for sweep speed error, displacement error and transmission error.

(OR)

23.	Explain the principle and operation of transistorized miller time base generator.

24.	Explain the operation of base timing Monostable blocking oscillator and neatly sketch its waveforms.

(OR)

25.	Write short notes on

a.	Unidirectional diode gate														(8)

b.	Bidirectional diode gate														(7)