# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	ELECTRONIC CIRCUITS 								       Time: 3 hours

# Subject Code:	09EE201	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define efficiency.

2.	What is the formula for calculating ripple factor?

3.	A low pass RC circuit behaves as a differentiator. True / False.

4.	What is an integrator?

5.	What is CMRR?

6.	Mention any one application of RC coupled amplifier.

7.	What is class B amplifier?

8.	What is loop gain?

9.	In a RC phase shift oscillator, if R1=R2=R3= 200k and C1=C2=C3= 100pf, find the frequency of oscillations.

10.	Which wave shaping circuit is used for generation of square wave from a sine input?

PART – B (5 x 3 = 15 MARKS)

11.	A half wave rectifier is used to supply 24V DC to a resistive load of 500 ohms and the diode has forward resistance of 50 ohms. Calculate the maximum value of ac voltage required at the input.

12.	What is the two level slicer circuit? Draw its circuit.

13.	Draw the characteristics of CE amplifier and CC amplifier.

14.	What is harmonic distortion in amplifiers?

15.	Draw the general form of LC oscillator and its equivalent circuit.

PART – C (5 x 15 = 75 MARKS)

16.	Derive an expression for ripple factor of an inductor filter.

(OR)

17.	Explain the operation of switched mode power supply.

18.	Obtain the response of high pass RC circuit for sinusoidal, step, pulse and square inputs.

(OR)

19.	Explain about pulse transformers and its equivalent circuits.

20.	Perform the common mode analysis of differential amplifier using BJT.

(OR)

21.	Describe the operation of direct coupled amplifiers.

22.	What are class B amplifiers? Explain the operation of class B push-pull amplifiers.

(OR)

23.	The current-series feedback type transistor amplifier has R1=R2=20k, hie=2k, RL= 1k, Re=100ohms, hfe=80. Calculate A, β, Rif, Af and loop gain in dB.

24.	Derive the frequency of oscillations for colpitts oscillator.

(OR)

25.	With a neat diagram, explain the operation of monostable multivibrator.