# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	NETWORK ANALYSIS AND SYNTHESIS 				       Time: 3 hours

# Subject Code:	EE203		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	List out any two properties of an Incidence matrix.

2.	Write the condition for reciprocity for a given network.

3.	Determine the poles and zeros of the impedance function: 	Z(s)   =   s2+4s+3/ (s3+6s2+8s)

4.	The cut-set schedule gives the relation between _________ and _________

5.	List out any two properties of a Hurwitz polynomial.

6.	For a two port bilateral network, the three transmission parameters are given by A=6/5; B=17/5; C=1/5. What is the value of D?

7.	List any two advantages of constant K filters.

8.	What is the importance of composite filters?

9.	Why are Foster and Cauer forms of networks often termed as canonic forms?

10.	What are the two different elements of realisability?

PART – B (5 x 3 = 15 MARKS)

11.	A function is given as z(s) = 3(s+1)/(s(s+2) (s+3)). Find the final value of z(t) by final value theorem.

12.	Write down the stability criterion for an active network.

13.	Derive the relation between Twigs and Links for any given tree.

14.	Compute the h parameters from the following data.

a.	with output terminals short circuited; V1=25V, I1= 1A, I2= 2A

b.	with input terminals open circuited; V1= 10V, V2= 50V, I2= 2A

15.	Check the positive realness of the following function. Z(s) =   (s+3)/(s+1)

PART – C (5 x 15 = 75 MARKS)

16.	A circuit is known to have its transfer function as H(s) = (s+3)/ (s2+4s+5) Find its output when:

a.	the input is a unit step function		b.	the input is 6te-2tu(t)

(OR)

17.	A series connected circuit has R=4 ohms and L=25mH. (a) Calculate the value of C that will produce a quality factor of 50. (b) Find ω1, ω2 and B. (c) Determine the average power dissipated at ω=ωo, ω1, ω2. Take Vm=100V.

18.	Draw the pole zero diagram for the given network function I(s) and hence obtain the i(t)

I(s)    =    20s/ ((s+5) (s+2))

(OR)

19.	Construct the bode plots for the transfer function: H () = 200j/ ((j+2) (j+10))

20.	What is fundamental cut-set? Explain the procedure of forming fundamental Cut-Set matrix?

(OR)

[P.T.O]

21.	For the figure given below, which represents a resistive circuit; determine the number of branches, number of nodes and number of links. Write down the incidence matrix for the given network. Also develop the network. Also develop the network equilibrium equation.

22.	Measurements were made on a typical two terminal network.

a.	With terminal pair 2 open, a voltage of 100 volts is applied to terminal pair 1 resulted in I1= (10+j0) Amps and V2= (25+j0) volts.

b.	With terminal pair 1 open, same voltage applied to terminal pair 2, resulted in I2= (20+j0) Amps and V1= (50+j0) volts.

c.	Write the loop equations for this network and also find the driving point and transfer impedances.

d.	What will be the voltage across a 10 ohms resistor connected across terminal pair 2 if (100+j0) volts source is connected across terminal pair 1?

(OR)

23.	Design a m-derived low pass filter having cut-off frequency of 1 kHz, design Impedance of 400 ohms and the resonant frequency 1100Hz.

24.	The driving point impedance of a one port reactive network is given by

Z(s)    =    5 ((s2+4) (s2+25)/(s (s2+16))).   Obtain the first and second Foster networks.

(OR)

25.	The driving point impedance of an LC network is given by Z(s) = s4+4s2+3/ (s3+2s). Determine the second Cauer form of the network.