# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	ELECTROMAGNETIC FIELDS 						       Time: 3 hours

# Subject Code:	EE204		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define Surface Charge Density.

2.	Write down the expression for Energy stored in a Capacitor.

3.	Why div. B is Zero?

4.	What is meant by Magnetic Flux?

5.	State Lenz’s Law.

6.	What is the significance of Divergence Theorem?

7.	Define Brewster angle.

8.	Write down the expression for Complex Poynting Vector.

9.	What are Boundary Conditions?

10.	What is meant by FEM and FDM?

PART – B (5 x 3 = 15 MARKS)

11.	State and Explain Coulomb’s Law.

12.	Explain the Boundary Conditions for Magnetic Field Vector.

13.	Discuss the concept of Eddy Current.

14.	Define Skin effect and depth of penetration.

15.	Write a note on Charge Simulation Method.

PART – C (5 x 15 = 75 MARKS)

16.	Derive an expression for Electric Potential and Field due to a Electric Dipole.

(OR)

17.	Find the Capacitance for a Coaxial Cable with inner radius ‘a’ and outer radius ‘b’ with length ‘L’.

18.	State and Explain Biot – Savart’s Law. Find an expression for Magnetic Field due to a line conductor using Biot – Savart’s Law.

(OR)

19.	a.	State and Prove Ampere’s Circuital Law.

b.	Calculate the Force / Unit Length between two conductors, carrying current of 5 A each in opposite direction separated by a distance of 1 m in free space.

20.	Derive Maxwell’s Equation in Integral Form and Differential Form. State the basic laws from which they are derived.

(OR)

21.	Discuss in detail, the relation between Field Theory and Circuit Theory.

[P.T.O]

22.	Derive the Wave equations for Electromagnetic Wave Propagation through free space.

(OR)

23.	Derive the Poynting Vector and show how the power flow in the form of E and H fields.

24.	Explain the finite difference method. Show how this method is applied to find potential at a point where potentials at four surrounding equidistant points are known in the presence of charge density ρ in x, y co-ordinates.

(OR)

Y

X

1

2

3

4

25.	Consider the two - element mesh shown below using FEM method, determine the potentials within the mesh.

| Node | (x, y) |
| --- | --- |
| 1 | (0.8, 1.8) |
| 2 | (1.4, 1.4) |
| 3 | (2.1, 2.1) |
| 4 | (1.2, 2.7) |