# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	DC MACHINES AND TRANSFORMERS 				       Time: 3 hours

# Subject Code:	EE207		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What are the methods of excitation used in DC Generator?

2.	What are the conditions for parallel operation of two DC Generators?

3.	Draw the speed torque characteristics of DC shunt motor.

4.	What are the methods used to control the speed of DC motor?

5.	Define regulation of single phase transformer.

6.	Write emf equation of single phase transformer.

7.	Define All day efficiency.

8.	What are the conditions for parallel operation of two three phase transformers?

9.	Define turns ratio of PT.

10.	What are the applications of CT?

PART – B (5 x 3 = 15 MARKS)

11.	Discuss no load and load characteristics of DC shunt generator.

12.	Obtain torque equation of DC motor.

13.	Draw and explain no load phasor diagram of transformer.

14.	What are the losses in single phase transformer?

15.	Define ratio and phase angle error in PT.

PART – C (5 x 15 = 75 MARKS)

16.	With neat sketch explain construction and principle of operation of DC generator.

(OR)

17.	Describe with sketch the effect of armature reaction in DC generator.

18.	Explain how to predetermine efficiency characteristics of DC generator by Swinburne’s test.

(OR)

19.	The Hopkinson’s test on two shunt machines gave the following results for full load: Line voltage = 250V; current taken from the supply system excluding field Currents = 50A: motor armature currents = 380A: field currents 5A and 4.2A. Calculate the efficiency of the machine working as a generator. Armature resistance of each machine is 0.2 ohm.

20.	Explain construction and principle of operation of single phase transformer.

(OR)

21.	Obtain approximate equivalent circuit of single phase transformer.

22.	Explain Sumpner’s test with circuit diagram and discuss how to calculate efficiency of transformer.

(OR)

[P.T.O]

23.	Two 100kW single phase transformers are connected in parallel both on the primary and secondary. One transformer has an ohmic drop of 0.5% at full load and an inductive drop of 8% at full load. The other has an ohmic drop of 0.75% and inductive drop of 2%. Show how they will share a load of 180kw at 0.9 power factor.

(OR)

24.	Explain constructional features of three phase transformer and describe how to put Parallel operation.

(OR)

25.	Describe constructional features and operation of current   transformer.