# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	SYNCHRONOUS AND INDUCTION MACHINES		       Time: 3 hours

# Subject Code:	EE209		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define pitch factor.

2.	Define distribution factor.

3.	What are the conditions for parallel operation of two alternators?

4.	Write the expression for synchronizing torque when alternator is connected to infinite bus bars.

5.	What are the steps for starting of a synchronous motor?

6.	The rotor of the synchronous motor always runs at _______ speed.

7.	 Draw slip-torque characteristics of Induction motor.

8.	What are the methods to control speed of Induction motor?

9.	Draw the diagram of spilt phase single phase Induction motor.

10.	Draw the diagram of shaded pole single phase Induction motor.

PART – B (5 x 3 = 15 MARKS)

11.	Obtain expression for emf equation of an alternator.

12.	Derive the expression for synchronizing power when two alternators are connected in parallel.

13.	What is hunting?

14.	Draw the equivalent circuit of three phase Induction motor.

15.	What is the basic principle of working of linear Induction motor?

PART – C (5 x 15 = 75 MARKS)

16.	With neat circuit, explain how to predetermine regulation of three phase alternator by synchronous impedance method.

(OR)

17.	Describe how to obtain regulation characteristics of alternator by Potier method.

18.	a.	Obtain expression for synchronizing power when two alternators are connected in parallel.																						(7)

b.	A 750 kVA, 11 kV , 4 pole 3 phase star connected alternator has percentage resistance and reactance of 1 and 15 respectively. Calculate the synchronizing power per mechanical degree of displacement at (i) no load (ii) at full load 0.8 p.f. lag. The terminal voltage in each case is 11kV. 																			(8)

(OR)

19.	Explain two reaction theory with complete phasor diagram of salient pole synchronous machine and obtain expression for regulation.

20.	a.	Explain construction and principle of operation of synchronous motor. 					(7)

b.	Describe synchronous motor on load with constant excitation phasor diagram.			(8)

(OR)

[P.T.O]

21.	a.	Explain the effect of excitation of an armature current and power factor of synchronous 			motor. 																			(7)

b.	Explain how to obtain V and inverted-V curves of  synchronous motor.					(8)

22.	Explain construction and principle of operation of three phase Induction motor and obtain expression for maximum torque.

(OR)

23.	A 3 phase 400V Induction motor gave the following test reading No load: 400V, 1250W, 9A. Short circuit: 150V, 4kW, 38A. Draw the circle diagram. If the normal rating is 14.9 kW, find the circle diagram, the full load value of current, p.f and slip.

24.	Explain double revolving field theory for the starting of single phase induction motor.

(OR)

25.	a.	Describe constructional features and operation of capacitor start and run motor.			(7)

b.	Obtain the equivalent circuit of single phase induction motor. 							(8)