# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	ELECTRICAL MACHINE DESIGN 						       Time: 3 hours

# Subject Code:	EE211		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Write down the formula for computing the mmf ofr air-gap length.

2.	How can the design problems of an electrical machine be classified?

3.	Write the expressions for gap contraction factor for slots and ducts.

4.	Define slot insulation factor.

5.	What are dummy coils?

6.	How will you select the back pitch and front pitch for a wave winding?

7.	What are the main classifications of transformer winding?

8.	Give typical values of Kc for various types of transformers.

9.	How can the induction motor be designed for best power factor?

10.	What are the methods adopted to reduce harmonic torques?

PART – B (5 x 3 = 15 MARKS)

11.	What are the major considerations to evolve a good design of electrical machine?

12.	Calculate the output coefficient of a dc shunt generator from the given data.

Bg = 0.89 Wb/m2, ac = 32000 amp.cond./m,  ψ = 0.66

13.	Mention the factors that govern the choice of number of armature slots in a dc machine.

14.	Write down the output equation of 1-Φ and 3-Φ transformers.

15.	List the various values of L/τ used for separation of D and L in induction motor.

PART – C (5 x 15 = 75 MARKS)

16.	Calculate the specific electric and magnetic loading of  120 HP, 3000 V, 3-phase, 50 Hz, 8-pole, star connected, induction motor having stator core length 0.6 m and stator bore = 0.66 m. Turns per phase = 286. Assume full load efficiency as 0.95 and power factor as 0.85

(OR)

17.	a.	What are the items to be mentioned in the rating plate of rotating machinery?

b.	Write a short note on standard specifications.

18.	a.	Calculate the mmf required for air-gap of a dc machine with an axial length of 25cm (no 	ducts) and pole arc of 20 cm. The slot pitch = 25 mm, slot opening = 12 mm, air gap = 6 	mm and the useful flux per pole = 25 m Wb. Take carter’s coefficient for slot as 0.3.

b.	Find the permeability at the root of the teeth of a dc machine armature from the following data: slot pitch = 2.2 cm, tooth width at the root = 1.07 cm, gross length = 32 cm, stacking factor =0.9, real flux density at the root of the teeth = 2.34 tesla, apparent flux density at the root = 2.46.

(OR)

19.	a.	How is the mmf of a section of magnetic circuit determined?

b.	How is the variable cross-section of the tooth accounted for in determining ampere turns 	for tooth?

[P.T.O]

20.	Draw the winding diagram in a developed form for a 4-pole, 13 slot simplex wave connected dc generator with a commutator having 13 segments. The number of coil sides per slot is 2. Indicate the position of brushes.

(OR)

21.	Calculate the main dimensions of 25 HP, 1000 rpm, 400 V, dc motor. Given that

Bav=0.35Wb/ m2 and ac = 16000 amp.cond./mt. Assume an efficiency of 91%.

22.	Determine the dimensions of core and window for a 5 kVA, 50 Hz, 1-phase, core type transformer. A rectangular core is used with long side twice as long as short side. The window height is 3 times the width.   Voltage per turn = 1.8 V. Space factor = 0.2, δ = 1.75 A / mm2.

Bm = 1 Wb / m2.

(OR)

23.	a.	List the different methods of cooling of transformers.

b.	The voltage per turn of a 500 kVA, 11 kV/ 415 V, Δ / Υ, 3-phase transformer is 8.6 V. Calculate the number of turns per phase of low voltage and high voltage windings.

24.	a.	Write down the concept of Electro mechanical system modeling for designing in electrical 	machines.

b.	What are ranges of efficiency and power factor in induction motor?

(OR)

25.	Design a cage rotor for a 40 HP, 3-phase, 400 V,50Hz,6-pole delta connected induction motor having a full load efficiency of 86% and full load power factor of 8.85. Take D= 35cm and L=17cm. Stator slots =54, conductors per slot =14. Assume suitably the missing data if any.