# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	POWER ELECTRONICS 								       Time: 3 hours

# Subject Code:	EE213		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define latching current.

2.	Name the power semiconductor device having highest switching speed.

3.	What is meant by commutation?

4.	What is meant by firing angle?

5.	What is the need Zero Crossing Detector in a control circuit?

6.	What is the relationship between the input and output voltage of step up chopper?

7.	Mention the types of DC-AC converters with respect to power circuit configuration.

8.	What is the condition for obtaining sustained oscillations in a series inverter?

9.	Mention the applications of AC voltage controllers.

10.	What are advantages of SMPS over linear power supply?

PART – B (5 x 3 = 15 MARKS)

11.	Draw the VI characteristics of SCR.

12.	Distinguish between symmetrical and asymmetrical configurations of single-phase half wave phase controlled AC-DC converter.

13.	Draw the block diagram of the firing circuit for a chopper.

14.	What is a cycloconverter? Enumerate some of its industrial applications.

15.	What is meant by Pulse Width Modulation?

PART – C (5 x 15 = 75 MARKS)

16.	a.	With basic structural diagram describe the steady state and dynamic characteristics of a 	power MOSFET.                                                                                                      		(8)

b.	Why does a GTO require high value of negative pulse during turn off? Also discuss how a negative pulse can be used to turn off a GTO.                                               				(7)

(OR)

17.	a.	Give the cross sectional views of a TRIAC and explain its turn on process with relevant 	diagram. Hence show that a TRIAC is rarely operated in first quadrant with negative gate 	current and in third quadrant with positive gate current.                    					(10)

b.	With a neat structural diagram explain the operation of a IGBT.                               		(5)

[P.T.O]

18.	a.	For a single phase fully controlled converter system, sketch waveforms for supply voltage, 	load voltage, load current and voltage across the SCR for i)RL load and ii)RL load with 	freewheeling diode across RL. From a comparison of these waveforms, discuss the advantages 	of using freewheeling diode.                              										(7)

b.	A single phase fully controlled converter is supplied from a 230V, 50Hz source.   The load consists of R = 10  and a large inductance so as to remains the load current constant.  For a firing   angle delay of 30, determine (i) average output voltage (ii) average output   current (iii) average and RMS values of SCR current and (iv) the power factor. 					(8)

(OR)

19.	a.	Explain the operation of a three phase six pulse converter feeding resistive load with neat 	waveforms.                                                                                                       				(10)

b.	Discuss the effect of source inductance on the performance of AC-DC converters.		(5)

20.	a.	With neat power circuit diagram explain the operation of a step down chopper.     		(8)

b.	Explain the operation of load commutated Chopper?                                              		(7)

(OR)

21.	a.	Describe the operation of voltage commutated Chopper.	                                      		(7)

b.	Explain the operation of a four quadrant chopper.                                                   		(8)

22.	Discuss the principle of working of a three phase bridge inverter with an appropriate circuit diagram. Draw phase and line voltage waveform on the assumption that each IGBT conducts for 120o and the load is star connected. The sequence of firing of various IGBTs should also be indicated in the diagram.

(OR)

23.	a.	Describe the working of a single phase basic series inverter with appropriate circuit and 	waveforms. What are the limitations of this basic series inverter?                     			(8)

b.	Describe the single phase full bridge inverter with the PWM method.                    		(7)

24.	a.	Explain the operation of a single phase, bidirectional AC voltage controller feeding unity 	power factor load with neat waveforms. Also derive the expression for input power factor.(9)

b.	A single phase ac voltage controller with anti parallel thyristor feeds a resistive load of 7.5     Ω. The supply voltage of the controller is 230V at 50Hz. The thyristors are fired at an angle of α=60o. Determine i) the rms value of the output voltage ii) the input power factor. 		(6)

(OR)

25.	a.	Discuss why three phase to single phase cycloconverter requires positive and negative 	group phase controlled converters? Under what conditions, the groups work as rectifiers or 	inverters? How should the firing angles of the two converters be controlled? 			(10)

b.	With neat block diagram explain the operation of SMPS.                                        		(5)