# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	PROTECTION AND SWITCH GEAR 					       Time: 3 hours

# Subject Code:	EE219		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Can a relay, having a time – current characteristics steeper than extremely inverse relay be realized using electromagnetic construction?

2.	Define pick up value of an electromagnetic relay.

3.	To co-ordinate over current relays for transmission line protection, the method of discrimination preferred is ____________ discrimination.

4.	Alternator differential protection does not operate for ____________ fault.

5.	Under what condition, the arc re-strikes in a circuit breaker?

6.	Define transient recovery voltage.

7.	Would you prefer SF6 circuit breaker or air blast circuit breaker for modern high voltage substations?

8.	Current zero interruption is applicable only in ____________ circuit breaker.

9.	Mention any two causes for over voltage occurrence in overhead transmission and distribution systems.

10.	A series spark-gap is not essential in ____________ surge arrester.

PART – B (5 x 3 = 15 MARKS)

11.	Explain the need for protective relaying in power systems.

12.	What are the uses of Buchholz’s relay?

13.	Explain Cassie’s theory of arc interruption.

14.	What are the major duties of circuit breakers?

15.	List out the ideal characteristics of surge arrester.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Discuss the fundamental requirements of protective relaying. 						(5)

b.	Explain mho relay with neat sketches. Draw the R-X diagram.						(10)

(OR)

17.	Explain the construction and operation of directional over current relay with neat diagram and torque equation.

18.	Describe the role of CT’s and PT’s and their application in protective schemes.

(OR)

19.	With a neat sketch explain the differential system of protection applied to Star-Delta connected power transformer.

[P.T.O]

20.	a.	In a 132 kV system, the inductance and capacitance up to the location of the circuit breaker 	are 0.4 H and 0.015 μF respectively. Determine the maximum value of the re-striking 	voltage across the contacts of the circuit breaker, frequency of transient oscillation and 	maximum value of Rate of Re-striking Voltage.  									(9)

b.	Explain the phenomena of low inductive current chopping. 						(6)

(OR)

21.	What is resistance switching? Derive the expression for critical resistance in terms of system inductance and capacitance which gives no transient oscillation.

22.	Explain in detail about the SF6 circuit breaker and its limitations.

(OR)

23.	Describe the construction, operating principle and application of Vacuum circuit breaker. What are its advantages over conventional types of circuit breakers?

24.	a.	How will you protect the transmission lines against lightning strokes?				(10)

b.	What are the methods used for protection from over voltages? 						(5)

(OR)

25.	Describe the construction and working principle of Expulsion type lightning arrester and Valve type lightning arrester.