# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	ELECTRONIC CIRCUITS 								       Time: 3 hours

# Subject Code:	EE225		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The ripple factor of a power supply is a measure of:

a.	Its filter efficiency		b.	Its voltage regulation		c.	Diode rating

d.	Purity of power output.

2.	Electronic devices that convert DC power into AC power are called __________

3.	If two stages of a cascaded amplifier have decibel gains of 60 and 30, then overall gain is ______dB.

4.	The gain of a Darlington amplifier pair is determined by ______ the beta values.

5.	The common mode rejection ratio of an ideal differential amplifier is _______.

6.	The outstanding characteristics of a direct coupled amplifiers is its

a.	Utmost economy		b.	Temperature stability

c.	Avoidance of frequency-sensitive components

d.	Ability to amplify direct current and low frequency signals.

7.	What is the necessity of feedback in an amplifier?

8.	Closed loop gain of a feedback amplifier is the gain obtained when

a.	Its output terminals are closed		b.	Negative feedback is applied.

c.	Feedback loop is closed			d.	Feedback factor exceeds unity

9.	What is a relaxation oscillator?

10.	In an astable multivibrator,  = _________

PART – B (5 x 3 = 15 MARKS)

11.	Describe a basic DC power supply and half wave rectification.

Vcc = +10V

12.	Determine Ic in the voltage divider biased transistor circuits if dc =100; R1=10k; R2 = 5.6k; Rc =1.0k; RE = 560;

Rc

1.0 k

R1

10 k

R2

5.6 k

RE

560

13.	Describe the amplification properties of a FET.

14.	A Wien bridge oscillator is used for operation at f0 = 10 kHz.  Assuming R1 =R2= R; and C1= C2 = C; if the value of R is 100K, find the value of capacitor C?

[P.T.O]

15.	Determine the period and frequency of oscillation for an astable multivibrator with component values R1 =2k; R2 = 10k and C1=0.01 F and C2 = 0.05 F.

PART – C (5 x 15 = 75 MARKS)

16.	Draw the circuit diagram of half wave rectifier.  Explain its working principle and its operation  indicating the frequency of ripple in its output.

(OR)

17.	Design the Zener regulator for the following specifications: output voltage = 5V; load current = 20 mA; zener power dissipation Pz(max) = 500mW; input voltage = 123 V.

18.	With the help of a suitable circuit diagram, explain the working of an RC coupled amplifier. Derive the expression for voltage gain of the amplifier.

(OR)

19.	Explain with a neat diagram a class A direct coupled amplifier and explain clearly its operation.

20.	With circuit diagram, A.C equivalent circuit and frequency response characteristics, explain the operation of a single tuned amplifier.

(OR)

21.	Explain in detail about the special features of differential amplifier.

22.	Explain the operation of the four types of feedback amplifier.

(OR)

23.	What is oscillator?  Explain in detail about the classification of oscillators with neat block diagrams and circuit diagrams.

24.	Explain in detail about the astable multivibrator and derive the expression for the period of square wave.

(OR)

25.	Explain in detail about the diode clampers and clippers circuits.