# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	CONTROL SYSTEMS 								      	       Time: 3 hours

# Subject Code:	EE237		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Name the two types of electrical analogous for the mechanical system.

2.	What are the three basic elements used in a closed loop control system?

3.	Write the standard test signals used in control systems.

4.	Define steady state error of a feedback control system.

5.	Define the term resonant peak of a closed-loop control system.

6.	What is phase cross-over frequency?

7.	What is characteristic equation of a control system?

8.	What is magnitude criterion in the root locus technique?

9.	Write the state model of linear time-invariant systems.

10.	State the condition for controllability by Kalman’s test.

PART – B (5 x 3 = 15 MARKS)

11.	Distinguish between open and closed loop control systems.

12.	What is the effect of PI controller on the system performance?

13.	Define the terms:  phase margin and gain margin.

14.	State Nyquist stability criterion.

15.	Write the properties of the state transition matrix.

PART – C (5 x 15 = 75 MARKS)

K1

B

M1

M2

K

B1

B2

x (output)

f(+) applied force

16.	Write the differential equations governing the mechanical system shown in figure and obtain the transfer function of the system.

(OR)

R(s)

G1

G2

G3

G4

G5

C(s)

G6

G7

-H1

-H2

17.	Consider the system shown in Figure. Obtain the closed-loop transfer function C(s)/R(s) by using Mason’s gain formula.

[P.T.O]

18.	A unity feedback system is characterized by an open-loop transfer function. Determine the gain K so that the system will have a damping ration of 0.5. For this value of K determine the settling time, peak overshoot and time to peak overshoot for a unit-step input.

(OR)

19.	The open-loop transfer function of a unity feedback control system is given by.  By applying Routh criterion, discuss the stability of the closed-loop system as a function of K. Determine the values of K which will cause sustained oscillations in the closed-loop system. What are the corresponding oscillation frequencies?

20.	Sketch the Polar plot of the transfer function given below.  Determine whether the plot cross the real axis. If so, determine the frequency at which the plot cross the real axis and the corresponding magnitude of G(jω).

(OR)

21.	Explain in detail about the determination of gain K for specified resonant peak, specified gain and phase margins using Nichol’s chart.

22.	A unity feedback control system has an open-loop transfer function.   Sketch the root locus plot of the system by determining the following

a.	Centroid, number and angle of asymptotes.

b.	Angle of departure of root loci from the poles.		c.	Breakaway point if any.

d.	The value of K and the frequency at which the root loci cross the jω axis.

(OR)

23.	An open-loop transfer function of the unstable is given by.  Using Nyquist stability criterion, determine whether the system is stable when the feedback path is closed.

24.	Derive state model for the system with closed loop transfer function	for which the system matrix A is diagonal.

(OR)

25.	Examine the observability of the system given by